Manually installing the Above Board software

Introduction

This appendix gives you the information you need to manually add the

necessary Above Board Plus and Plus I/O commands to your CONFIG.SYS file.

N O T E

This guide  assumes you know the differences between conventional,

extended, and expanded memory.  If you don't, refer to Chapter 2 in your

Above Board manual for definitions.

Manually Installing the Above Board software

The Above Board comes with software that lets you:

o use expanded memory

o use the conventional memory supplied by the Above Board as mappable

conventional memory

o create RAM disks

o create a print buffer  If you boot from floppy diskettes,

install the Above Board software on each system diskette from which

you boot.  If you boot from a hard disk, install the software just

once in the root directory.

Installing the software involves these steps:

o copying files

o setting up expanded memory

o setting up RAM disks

o setting up a print buffer

In general, you don't have to do all this copying and setting up.  The

SOFTSET program can do it automatically.  This appendix describes the

do-it-yourself technique.

Copying files

Before you can use the Above Board software, you must copy files from the

Intel diskette to your hard disk's root directory or to your system

diskettes.  Copy only the files you need. If you have more than one Above

Board software diskette, check the date on the diskettes, and make sure you

install the most recent version of the software.  The Above Board software

includes the following files:

File Name       Description

EMM.SYS         Activates expanded memory on

Above Board Plus or Plus I/O.

Also allows mappable conventional memory.

QUIKMEM2.SYS    Creates a RAM disk in expanded memory.

QUIKBUF2.COM    Creates a print buffer in expanded memory.

Copy the files you need for the features you want to use.

-1-

Setting up expanded memory for Above Board Plus and Plus I/O boards only

This section describes the command line you need to put into your

CONFIG.SYS file if Above Board Pluses or Plus I/Os provide all the expanded

memory in your computer.  If you will use both an Above Board Plus or Plus

I/O and another type of Above Board (Above Board  286, PS/286, AT, PS/AT,

PC, or PS/PC) to supply expanded memory, skip this section and go to page

F-19 for the appropriate EMM command line. To use expanded memory on Above

Board Plus and Plus I/O boards, add this command as the first line in your

CONFIG.SYS file:

DEVICE=EMM.SYS  Computer  I/O_Address 1..16 [Memory_Address]    [EXPF=x]

[EXPL=x]    [MC]    [MCF=x]    [MCL=x]    [RD]    [ND]    [NP]    [NE]

[VI]    [H=x]

The parameters without brackets ([]) are required; those in brackets are

optional.  The order of the parameters doesn't matter.

N O T E

The MC, RD, MCF, and MCL parameters work only with the Above Board Plus and

Plus I/O.

N O T E

If there is an Inboard PC installed in your system, the EMM.SYS command

line must be inserted below the INBRDPC.SYS line in the CONFIG.SYS file.

Required EMM command line parameters

The Computer parameter defines the

type of computer in which the Above Board is installed.  Replace the

Computer parameter with one of the choices listed here:

Parameter       Computer Type

pc (or PC)        COMPAQ DESKPRO

IBM PC

IBM XT

IBM PC or XT compatibles

at (or AT)        COMPAQ DESKPRO 286

COMPAQ DESKPRO 386

IBM AT

IBM XT 286

IBM PS/2 Model 30 286

IBM AT compatibles

mod30 (or MOD30)  IBM Personal System/2 Model 30

The I/O_Address parameters specify which base I/O addresses Above Boards

use.  Assign a unique I/O address to each Above Board.  No boards in your

computer can share I/O addresses.  Piggyback Memory boards use the I/O

addresses of the Above Boards to which they are attached.

Use one I/O_Address parameter value for each Above Board Plus and Plus I/O

in your system.  Be sure your I/O_Address parameter values match the I/O

addresses selected when you ran the SETBOARD program.  These are displayed

on the final summary screen when you run the SETBOARD program, or if you

choose the VIEW option in SETBOARD.

-2-

The possible values are:

I/O address

208

218

248

258

2A8

2B8

2E8

Example.  You're installing two Above Board Plus boards in an IBM AT.  One

board is set for I/O address 208, the other for address 258.  Use this

command:

DEVICE=EMM.SYS AT 208 258

Optional EMM command line parameters

The EXPF=x (expanded first) and EXPL=x (expanded last) parameters specify

the range of addresses of the expanded memory page frame.

If you omit the EXPF and EXPL parameters, EMM automatically searches the

address range between 768K and 960K (hexadecimal segment address C000 to

F000 is checked for ROM or RAM), selecting the largest unused contiguous

area in that range as the page frame.  The EXPF and EXPL parameters

override this automatic mapping and let you specify the range of the page

frame search.  This might be necessary to enable the Above Board to work

with some network and communication boards.

The EXPF parameter specifies the beginning of the first 16K-byte page.  The

EXPL parameter specifies the beginning of the last 16K-byte page.  The

values you use must be hexadecimal segment addresses that represent

16K-byte boundaries between 768K and 944K.  The range that you specify

should not overlap any RAM or ROM on other add-in boards in your computer.

The possible values are:

Value              Address

C000               768K

C400               784K

C800               800K

CC00               816K

D000               832K

D400               848K

D800               864K

DC00               880K

E000               896K

E400               912K

E800               928K

EC00               944K

You can include either or both of the EXPF and EXPL parameters.  If you

include both, EMM uses both addresses you specify and searches that range

for the largest unused contiguous area to use as the page frame (the

minimum size is 64K).

If you include just the EXPF parameter, EMM starts the page frame search at

the EXPF address and selects the largest contiguous area between that

address and 944K.

-3-

If you include just the EXPL parameter, EMM selects the largest contiguous

area it finds between 768K and the end of the 16K-byte area specified with

the EXPL parameter. If you specify addresses less than C000 or greater than

DC00, if you specify a page frame that is less than 64K, or if you invert

the values (that is, specify a larger value for EXPF than for EXPL), EMM

displays a message and does not install.

The MC (mappable conventional memory) parameter causes EMM to make all

conventional memory supplied by Above Board Plus or Plus I/O boards

mappable.  (You can limit the mappable range by using the MCF and MCL

parameters.)  Mappable conventional memory enables operating environments

such as Windows 2.0 and DESQview to perform conventional memory mapping.

Refer to Appendix E for more information about mappable conventional

memory. If you want to use conventional memory supplied by the Above Board

Plus or Plus I/O as mappable conventional memory (described in Appendix E),

be sure you do the following:

o       Make sure that you add the MC (or the MCF or MCL parameter)

to the EMM command line.

o       The Above Board must supply the conventional memory you want

to make mappable.  Many computers have system boards that

provide 512K or 640K of conventional memory, leaving either

128K or no memory that could be assigned as mappable.  If

you want more mappable conventional memory, disable some of

your system board memory.  See your computer's operation

guide for information on disabling some of the conventional

memory in your computer.  Appendix E has an example.

N O T E

Your computer must supply at least 256K bytes of conventional memory.

N O T E

Do not use the MC parameter if you have an Intel Inboard 386 installed in

your computer.

The MCF=x (mappable conventional first) and MCL=x (mappable conventional

last) parameters provide the same function as the MC parameter (making

conventional memory supplied by Above Board Plus and Plus I/O boards

mappable), but they also let you restrict the beginning and ending

addresses of mappable conventional memory.

These parameters are useful only if you run an operating environment (such

as Microsoft Windows 2.0) that uses version 4.0 of the Expanded Memory

Specification to map conventional memory.  If you include either or both of

the MCF and MCL parameters, you do not need to include the MC parameter.

If you don't want mappable conventional memory, you don't need to include

these parameters.

When you include the MC parameter, EMM determines the range of the

conventional memory supplied by Above Board Plus or Plus I/O boards (this

can start no lower than 256K) and this becomes the range of mappable

conventional memory.  If you use the MCF and MCL parameters instead of MC,

you can specify the starting and ending addresses for mappable conventional

memory within the address range supplied by the Above Board Plus or Plus

I/O board.

-4-

The MCF parameter specifies the beginning of the first 16K-byte area of

mappable conventional memory.  The MCL parameter specifies the beginning of

the last 16K-byte area of mappable conventional memory.  The values you use

must be hexadecimal segment addresses that represent 16K-byte boundaries

(starting at 256K) in the 640K-byte conventional memory area.  The possible

values are:

Value   Address            Value   Address

4000       256K            7000    448K

4400       272K            7400    464K

4800       288K            7800    480K

4C00       304K            7C00    496K

5000       320K            8000    512K

5400       336K            8400    528K

5800       352K            8800    544K

5C00       368K            8C00    560K

6000       384K            9000    576K

6400       400K            9400    592K

6800       416K            9800    608K

6C00       432K            9C00    624K

If you specify a value that is not a multiple of 16K, EMM rounds up the

value to the next 16K increment. You can include either or both of the MCF

and MCL parameters.  If you include both, EMM uses both addresses you

specify to determine mappable conventional memory range.

If you include just the MCF parameter, EMM starts mappable conventional

memory at the MCF address and ends it wherever the conventional memory

supplied by the Above Board ends.

If you include just the MCL parameter, EMM starts mappable conventional

memory wherever the conventional memory supplied by the Above Board starts,

and ends mappable conventional memory at the end of the 16K-byte area

specified with the MCL parameter.

If you specify addresses less than 256K or greater than 640K, or if you

invert the addresses (that is, specify a larger address for MCF than for

MCL), EMM displays a message and does not install.

Example 1.

You're using an IBM PS/2 Model 30 286 that includes one Above Board Plus,

set to I/O port 258.  You want to provide mappable conventional memory for

operating environments such as Microsoft Windows 2.0.  Your Above Board is

set to provide conventional memory from 256K to 640K.  You want EMM to

begin mapping at 256K and stop mapping at 624K (a requirement for Windows

and DESQview).  Use the following command:

DEVICE=EMM.SYS AT 258 MCL=9800

It is not necessary here to specify the starting address, since it will

automatically use 256K.  Notice that the MCL value (9800) is equivalent to

608K, not 624K.  That is because the MCL value represents the start of the

last 16K bytes of mappable memory.  Therefore, 608K + 16K = 624K, where

mappable conventional memory actually ends.

Because you did not specify EXPF and EXPL, EMM will automatically search

for and select an expanded memory page frame for you.

-5-

Example 2.

You're using an IBM XT that includes one Above Board Plus at I/O port 258.

The following command assigns all of the Above Board memory as expanded

memory. DEVICE=EMM.SYS PC 258

Because you did not include the EXPF and EXPL parameters, EMM automatically

uses the largest unused contiguous area between 800K and 960K as the page

frame.  The area selected depends on your system configuration.  Other

add-in boards with RAM or ROM in the address range between 800K and 960K

will limit the page frame size and location.  To load, EMM requires one

contiguous 64K-byte block in this range.

The following command instructs EMM to find a page frame between 800K and

896K:

DEVICE=EMM.SYS PC 258 EXPF=C800 EXPL=DC00

The RD (relocate driver) parameter causes EMM to be installed in expanded

memory instead of conventional memory.  This conserves conventional memory.

One page (16K) of expanded memory is reserved for use by EMM if you use

this parameter.

If the RD parameter is absent, EMM will be loaded into conventional memory.

The ND (no diagnostics) parameter tells software to run abbreviated memory

tests after a warm boot (Ctrl-Alt-Del).  The ND parameter is optional.  If

ND is in the command line, the EMM will run abbreviated memory tests; if ND

is absent, the EMM will run full tests.  In either case, full tests are run

during a cold boot (power-on).

The NP (no pause) parameter overrides the EMM pause after an EMM error or

advisory message.  The NP parameter is optional.  If NP is in the command

line, the EMM won't pause after errors or messages.  If NP is absent, the

EMM will pause.

The NE (no extended memory tests) parameter tells EMM not to run extended

memory tests.  Use this parameter in an IBM AT or compatible computer if no

extended memory exists on any Above Board or on your system board.

The NE parameter is optional.  If NE is in the command line, no extended

memory tests are performed.  If NE is absent, tests are performed.

C A U T I O N

Using the NE parameter can damage the data on your hard disk if you have

extended memory in your system.  Do not select this option if your Above

Board is supplying extended memory.

The VI parameter lets you cause all information on the EMM configuration to

be displayed.  It is optional.  If it is in the command line, all EMM

configuration information is displayed.  If VI is absent, only information

relating to the parameters you select is displayed.

The H=x (handle count) parameter lets you tell EMM to support as many

handles as a particular application program needs.  A handle is a value

that EMM assigns and uses to identify a block of expanded memory requested

by an application program.  EMM must keep track of all the handles it

supports; a large handle count increases the size of EMM.  Specifying a

small EMM handle count (less than 64) can save conventional memory and

allow EMM to run faster.

-6-

The EMM handle count default is 64 handles.  Application programs written

to use EMM versions below EMM 4.0 use a maximum of 32 handles.  If you're

using older application programs, you may change the EMM handle count to

H=32, but using the default of 64 handles will work fine.  The maximum value

for x is 255 handles. The H=x parameter is optional.  If the H=x parameter is

in the command line, the EMM will support the number of handles specified by

x.  The handle count will also appear on the screen when EMM is installed.

If H=x is absent, the EMM uses 64 handles.  You won't see a handle count on

the screen when EMM is installed unless you specify a value for the H

parameter or use the VI parameter in the command line.

Example.

You're using one Above Board Plus in an IBM AT.  You want expanded memory to

be installed at D000.  The board is set for I/O address 258.  You want all

conventional memory made mappable, and you want EMM installed in expanded

memory.  You want the handle count default of 64.  Use this command:

DEVICE=EMM.SYS AT EXPF=D000 258 MC RD

Set up when different Above Board types supply expanded memory

This section describes the command line you need to put into your

CONFIG.SYS file if you are using both an Above Board Plus or Plus I/O and

another type of Above Board (Above Board 286, PS/286, AT, PS/AT, PC, PS/PC)

to provide expanded memory in your computer. To use expanded memory on your

boards, add this command as the first line in your CONFIG.SYS file:

DEVICE=EMM.SYS   Computer   Memory_Address   I/O_Address 1...16    [EXPF=x]

[EXPL=x]    [ND]    [np]    [ne]    [vi]    [H=x] The parameters without

brackets ([]) are required; those in brackets are optional.  The order of

the parameters doesn't matter.  For explanations of these parameters, see

pages F-5 through F-18.  All the previously described parameters work

except MC, MCF, MCL, and RD -- these parameters work in computers that

contain the Above Board Plus or Plus I/O only.

N O T E

If there is an Inboard 386/PC installed in your system, the EMM.SYS command

line must be below the INBRDPC.SYS line in the CONFIG.SYS file.

Setting up RAM disks

You can set up Above Board RAM disks in expanded memory.  This section

explains how. For each RAM disk you install in expanded memory, put a line

like this in the CONFIG.SYS file after the EMM.SYS line:

DEVICE=QUIKMEM2.SYS x

The x parameter specifies the size of the RAM disk in K bytes and must be at

least 16.  Values that aren't multiples of 16 are rounded up to the

next-higher multiple of 16.  The maximum size of a RAM disk is 8M bytes

(8192K).

Example.

For two 1600K-byte RAM disks in expanded memory, put these two lines in the

CONFIG.SYS file: DEVICE=QUIKMEM2.SYS 1600#DEVICE=QUIKMEM2.SYS 1600 DOS

limits you to 26 disk drives, which includes all of the fixed and floppy

disk drives and RAM disks in the system.  It also assigns a unique drive

letter to each disk drive.  For a typical computer with a hard disk, the

first RAM disk is D.  For a two�floppy computer with no hard disk, the

first RAM disk is C.  Additional RAM disks receive sequential letters.

-7-

Setting up a print buffer

To set up a print buffer in expanded memory each time you turn on or

restart your computer, you'll need to add a command to your AUTOEXEC.BAT

file.  Use any ASCII file editor (such as EDLIN) to edit your file.

N O T E

You can have only one print buffer.  If a command in your AUTOEXEC.BAT file

already sets one up, delete that command before adding the commands

described in this section.  To set up the print buffer in expanded memory,

use this command:

QUIKBUF2  c  x  d  t

The c parameter tells the QUIKBUF program what type of computer you're

using.  The choices for the c parameter are:

Parameter                Computer Type

at (or AT)               IBM AT

IBM AT compatibles

IBM AT and XT/286

COMPAQ DESKPRO 286

pc (or PC)               IBM PC

IBM XT

IBM PC and XT compatibles

COMPAQ Deskpro 86

mod30 (or MOD30)         IBM Personal System/2 Model 30

mod30/286 (or MOD30/286) IBM Personal System/2 Model 30 286

The x parameter specifies the size of the buffer in K bytes and must be 32

or greater.  Any value that isn't a multiple of 16 is rounded up to the

next�higher multiple of 16.  16K of the buffer is reserved for the pop-up.

The d parameter tells the QUIKBUF program what display adapter your

computer uses. Choices for the d parameter are:

M (or m)                        IBM Monochrome Adapter.

H (or h)                        Hercules Monochrome Graphics#                   Adapter.

G (or g)                        IBM, Zenith, AT&T, or Hercules#                 Color Graphics Adapter.  IBM#                   Personal System/2 Model 30 #                    or COMPAQ monochrome or #                       color adapter.

E (or e)                        IBM Enhanced Graphics Adapter.

V (or v)                        IBM Video Graphics Array.#                      IBM Model 30 286 Adapter.

O (or o)                        All other adapters, including the#                      Hercules Graphics Plus Adapter.#                        Choosing this option disables the#                      print buffer command menu,#                     described in Chapter 13.  The print #                   buffer will still work, but you #                       won't be able to control it from#                       the command menu.

The t parameter indicates the type of port to be buffered.

Here are the choices:

P       Parallel  (LPT1, LPT2, or LPT3)

S1      Serial  (COM1)

S2      Serial  (COM2)

If you choose P, the first thing you print or the print buffer commands you

use will determine whether the buffered port is LPT1, LPT2, or LPT3.  See

the print buffer chapter in the manual for details.  If you omit the 't'

parameter, the print buffer defaults to parallel.

If you choose S1 or S2, you may need to add one or two MODE commands to

your AUTOEXEC.BAT file.  See the DOS manual or Chapter 15 in the Above

Board manual for details.

-8-

Example.  Suppose you have an IBM AT and an IBM Enhanced Graphics Adapter.

If you want a 144K-byte print buffer in expanded memory and you want to

print your data on a serial printer attached to COM1, use this command:

QUIKBUF2 AT 160 E S1

This creates a print buffer of 160K bytes, but since 16K is used for the

pop-up, you are left with the 144K-byte buffer you wanted. Using the

optional programs

This section describes the optional programs available on the Above Board

diskette.  These programs don't need to be run; they're included for your

convenience.

Here's how to start these programs:

1       With your screen displaying the DOS prompt, insert a copy of

the Intel diskette into drive A.

2       Type A:[command] and press Enter.

Replace the command parameter with the command listed in the

following table for the specific program you want to run.

Command         Program

CHKMEM          Displays the amount of conventional, extended, (IBM AT

and compatible computers only) and expanded memory in

your computer.

TESTAB          Tests and pinpoints bad RAM chips on your Above Board.

SOFTSET         Sets up expanded memory, mappable conventional memory,

RAM disks, and a print buffer.

-9-