Version 3.3 DigPak

written by

John W. Ratcliff

Includes one set of sources that works in all memory models.  Full

MIDPAK compatible support.

The set of digitized sound drivers hook into user interrupt vector 66h

to provide a clean clear programmers interface. This document describes

the two ways you can access the digitized sound package.  The first is

the interrupt level interface.	The second is a link library interface

that provides a set of C or assembly language callable functions to

access not only the digitized sound drivers but also provides usefull

support routines.  These glue code functions are located in the source

file DIGPLAY.ASM.

************** VERSION 3.1 CHANGES ***************

int far *ReportPendingAddress(void); // Reports the far address of the pending

// flag.	If this memory location pointed to by this address is 1 that means

// a pending sound effect is still pending.  When this becomes zero, then your

// application software can post the next piece of audio to play.  It is

// preferable to use this semaphore to know when to post the next buffer

// rather than to use the AudioPendingStatus call.

int far *ReportSemaphoreAddress(void); // Reports the far address of the DigPak

// semaphore.  If this semaphore is true, then DigPak is currently active

// and you shouldn't post any DigPak calls.  This is EXTREMELY important if

// you are trying to invoke DigPak functions via a hardware interrupt, where

// you could potentially have interrupted DigPak itself.

void far StopNextLoop(void); // If a digital sound sample is looping,

// this call will cause it to stop looping, the next time it has

// finished.

int far ReportVersionNumber(void); // Report the DigPak version number.

// Return code is times 100, meaning that version 3.1 would be returned

// as the decimal number 310.  This function wasn't supported prior to

// version 3.1 release, so it will be returned as 0, for versions prior

// to 3.1.

**************************************************

***********************************************************************

*************** INT 66h Interface Specification ***********************

***********************************************************************

When invoking digplay functions to play sound you pass the address of

a sound structure (SNDSTRUC) that contains the basic information

describing the sound effect you wish to play.

typedef struct

{

unsigned char far *sound; // Far address of audio data.

unsigned int	 sndlen;  // Length of audio sample.

int far *IsPlaying;	  // Address of play status flag.

int	frequency;	  // Playback frequency.

} SNDSTRUC;

********* DIGPAK functions ************************************

Function #1: DigPlay, Play an 8 bit digitized sound.

INPUT:	AX = 688h    Command number.

DS:SI	     Point to a sound structure that

describes the sound effect to be played.

Function #2: SoundStatus, Check current status of sound driver.

and report VERSION number of the driver, 3.1 and later.

INPUT:	AX = 689h

OUTPUT: AX = 0	     No sound is playing.

= 1	     Sound effect currently playing.

DX = 0	     No sound is looping.

= 1	     A sound effect is looping.

BX = version Starting with version 3.1, the BX register

of the SoundStatus call will return the

version number.  The version number is in

decimal, and multiplied times 100.  Meaning

a return of 310, is equal to version 3.10.

Versions before 3.1, did not set the BX

register to anything, so you should zero

out the BX register before you check the

version number.  If the BX register is still

zero, then the DigPak driver loaded is less

than 3.1.

Function #3: MassageAudio, Preformat audio data into ouptut hardware format.

INPUT:	AX = 68Ah

DS:SI	     Point to address of sound structure.

Function #4: DigPlay2, Play preformatted audio data.

INPUT:	AX = 68Bh

DS:SI	     Point to address of sound structure.

Function #5: AudioCapabilities, Report capabilities of hardware device.

INPUT:	AX = 68Ch

OUTPU:	AX = Bit 0 -> On, supports background playback.

Off, driver only plays as a foreground process.

Bit 1 -> On, source data is reformatted for output device.

Off, device handles raw 8 bit unsigned audio.

Bit 2 -> On, Device plays back at a fixed frequency, but

the audio driver will downsample input data

to fit.

Off, device plays back at user specified frequency.

(NOTE: You can still playback an audio sample at

whatever frequency you wish.  The driver

will simply downsample the data to fit

the output hardware.  Currently it does

not support upsampling though.)

Bit 3 -> On, this device uses the timer interrupt vector

during sound playback.

Bit 4 -> Driver supports timer sharing.

Bit 5 -> Supports looped sounds and pending.

Bit 6 -> Supports stereo panning.

Bit 7 -> Supports 8 bit PCM stereo playback.

Bit 8 -> Supports audio recording.

Bit 9 -> supports DMA backfilling.

DX = If this device plays back at a fixed frequency the DX

register will contain that fixed frequency playback rate.

Function #6: ReportSample, Report current playback address. UNSUPPORTED CALL

THIS FUNCTION CALL WAS DESIGNED BUT NOT ABLE TO BE IMPLEMENTED

FOR ALL DRIVERS!!

INPUT:	AX = 68Dh

OUTPUT: AX = Current playback address. UNSUPPORTED FUNCTION CALL!!!

Function #7: SetCallBackAddress, sets a user's sound completion

callback addess.

INPUT: AX = 068Eh

BX = Offset portion of far procedure to callback.

DX = Segment portion of far procedure to callback.

DS = Data Segment register value to load at callback time.

OUTPUT: None.

This function allows the user to specify a callback

address of a far procedure to be invoked when a sound

effect has completed being played.  This function is

disabled by default.  Sending a valid address to this

function will cause a callback to occur whenever a sound

sample has completed being played.  The callers DS register

will be loaded for him at callback time.  Be very careful

when using this feature.  The application callback procedure

is being invoked typically during a hardware interupt.

Your application should spend a small an amount of time

as possible during this callback.  Remember that the

callback must be a far procedure.  The sound driver

preserves ALL registers so your callback function does

not need to do so.  Do not perform any DOS functions

during callback time because DOS is not re-entrent.

Keep in mind that your own application has been interupted

by the hardware it this point.  Be very careful when making

assumptions about the state of your application during

callback time.  Hardware callbacks are generally used

to communicate sound event information to the application

or to perform a technique called double-buffering, whereby

your application immediatly posts another sound effect to

be played at the exact time that the last sound effect

has completed.

WARNING!!! Be sure to turn off hardware callbacks when

your application leaves!!! Otherwise, harware callbacks

will be pointing off into memory that no longer contains

code.  This function is for advanced programmers only.

Function #8: StopSound, stop currently playing sound.

INPUT: AX = 68Fh

OUTPUT: None.

Will cause any currently playing sound effect to be

terminated.

Function #9: SetAudioHardware, set up hardware information. UNSUPPORTED

FUNCTION CALL!!!!!!

This function is not available.  A new method is now

available for reconfiguring the hardware setup for

any driver.  Every driver is a file on disk, in .COM

format.  At byte locations 106h, 108h, and 10Ah, you

will find the following values:

Base Hardware Address: 106h offset in file.

IRQ:		       108h

Other (dma channel):   10Ah

If any of these three locations contain a -1, then that

value cannot be reconfigured.  If all three contain a -1,

then nothing can be reconfigured for this driver.  Otherwise

it contains the default value for this driver.	Your setup

program can poke new values into these locations, according

to the configuration information provided by the user.

FUNCTION #10: ReportCallbackAddress

INPUT: AX = 691h

OUTPUT: AX:DX -> far pointer to current callback address.

BX -> original caller's DS register.

This function should probably never need to be used by your

application software.  It is provided because the MIDPAK,

MIDI driver, needs to revector hardware callbacks so that

it can handle hardware contention problems between digitized

sound playback and synthesized sound playback.

FUNCTION #11: RestoreHardware NOT TO BE USED BY APPLICATION SOFTWARE!!!

INPUT: AX = 692h

OUTPUT:

Put hardware back to initial state.  Invoked by the

DeInstall code.  Not to be called by an application program!

FUNCTION #12: SetTimerDivsorRate

INPUT: AX = 693h

DX = Countdown timer divisor rate, so that timer based

drivers can service application timer interrupts

at their previous rate.  Service rate will be an

aproximation, that is fairly close.  To reset timer

divisor to default of 18.2 pass a 0 in the DX register.

WARNING!!! If you make use of timer sharing, be sure

to reset it back to the normal 18.2 time per second

rate when you exit.

FUNCTION #13: DigPlayLoop (not a MIDPAK compatible function)

This function is in beta form, for version 2.1 release.

INPUT: AX = 694h

DS:SI ->sound structure, preformated data.

FUNCTION #14: PostAudioPending

INPUT: AX = 695h

DS:SI -> sound structure, preformated data.

OUTPUT: AX = 0	Sound was started playing.

AX = 1	Sound was posted as pending to play.

AX = 2	Already a sound effect pending, this one not posted.

FUNCTION #15: AudioPendingStatus

INPUT:	AX = 696h

OUTPUT: AX = 0 No sound is playing.

AX = 1 Sound playing, sound pending.

AX = 2 Sound playing, no sound pending.

FUNCTION #16: SetStereoPan

INPUT: AX = 697h

DX = stereo pan value. 0 full volume right.

64 full volume both.

127 full volume left.

OUTPUT: AX = 0 command ignored, driver doesn't support stereo panning.

AX = 1 pan set.

FUNCTION #17: SetPlayMode

INPUT: AX = 698h

DX = Play Mode function.

DX = 0 -> 8 bit PCM

= 1 -> 8 bit Stereo PCM (left/right)

= 2 -> 16 bit PCM

= 3 -> 16 bit PCM stereo

Once the play mode has been changed, all DigPak functions

work exactly the same as before but expect data passed as

specified by this play mode.  8 bit PCM Stereo has left/right

channel pairs.	Supported by Stereo FX card and Sound Blaster

Pro.

OUTPUT: AX = 1 -> mode set.

AX = 0 -> mode not supported by this driver.

FUNCTION #18:	Report Pending Status Flag adddress and Semaphore address.

int far *ReportPendingAddress(void);

Reports the far address of the pending flag.  If this memory location

pointed to by this address is 1 that means a pending sound effect is

still pending.	When this becomes zero, then your application software

can post the next piece of audio to play.  It is preferable to use this

semaphore to know when to post the next buffer rather than to use the

AudioPendingStatus call.

int far *ReportSemaphoreAddress(void);

Report the address of the DigPak semaphore.  This returns a far

address to a word location inside DigPak.  When this is true,

then DigPak is currently active, and you shouldn't invoke any

DigPak function from a hardware interrupt at this time.

INPUT: AX=699h

OUTPUT: AX:DX -> form far Address of Pending status flag.

BX:DX -> form far address of digpak semaphore

FUNCTION #19: Set audio recording mode.

INPUT: AX = 69Ah

DX = 0 turn audio recording ON.

= 1 turn audio recording OFF.

OUTPUT: AX = 0 sound driver doesn't support audio recording.

AX = 1 audio recording mode is set.

FUNCTION #20:	StopNextLoop

INPUT:	AX = 69Bh

OUTPUT: NOTHING

If the DigPak driver is currently looping a sound

effect, then that loop will be cancelled, the next time

around.

FUNCTION #21: Set DMA back fill mode.

INPUT: AX = 69Ch

DX = backfill mode 0 means turn it off.

and a 1 means to turn it off.

OUTPUT: AX = 1 -> back fill mode set.

0 -> driver doesn't support DMA backfill.

FUNCTION #22: Report current DMAC count.

INPUT: AX = 69Dh

OUTPUT: AX = Current DMAC count.

FUNCTION #23: Verify DMA block, check to see if it crosses a 64k page

boundary for the user.

INPUT: AX = 69Eh

ES:BX -> address of sound.

CX    -> length of sound effect.

OUTPUT: AX = 1 Block is ok, DOESN'T cross 64k bounadary.

AX = 0 block failed, DOES cross 64k boundary.

FUNCTION #24: Set PCM volume.

INPUT: AX = 69Fh

BX = Left channel volume (or both if mono) 0-100

CX = Right channel volume (or both if mono) 0-100

OUTPUT: AX = 1 Volume set

AX = 0 Device doesn't support volume setting.

***********************************************************************

** DigPlay, linkable interface to the Digitized Sound Package.	  *****

***********************************************************************

The file DIGPLAY.ASM is a small assembly language source file that

provides C compatible procedure calls to hook into the Digitized

Sound Package.	DIGPLAY.ASM uses one include file called PROLOGUE.MAC.

This is a macro header file that defines some basic useful assembly

language macros.

DIGPLAY.ASM is written in Turbo Assebmler's IDEAL mode.  The MASM syntax

for 8086 assembler is abhorent.  The Turbo Assembler IDEAL mode offers

true local labels, real data structures, typecasting, automatic argument

passing, and local memory allocation. Turbo Assembler in IDEAL mode

provides many of the features of a high-level langauge.  Turbo Assembler

and Turbo Debugger combined are without question the most powerful

assembly language tools available for any 8086 programmer.  The last I

checked, Turbo Assembler and Turbo Debugger sold for less than $99.

Recently Microsoft released a new version of MASM that finally provides

some of the features that Turbo Assembler has had for over three years;

and requires a new assembly language syntax.  For these reasons I can

no longer consider MASM the standard for 8086 DOS based assemblers and

that it why you will find all assembly language source code here

provided in Turbo Assembler IDEAL mode format.	If you do not have

Turbo Assembler simply link to one of the two following object modules.

All these link layer procedures are defined as far calls and accept far

addresses for all pointers passed.  The reason for this is so that you

can link to one object module regardless of the memory model you are

working in.

The C prototype header file for DigPlay support functions are as follows:

#ifndef LOADABLE_DRIVERS

#define LOADABLE_DRIVERS 1 // Conditional compilation, set to true if

// Supporting loadable drivers.

#endif

/* Bit flags to denote audio driver capabilities. */

/* returned by the AudioCapabilities call.				*/

#define PLAYBACK 1    // Bit zero true if can play audio in the background.

#define MASSAGE  2    // Bit one is true if data is massaged.

#define FIXEDFREQ 4   // Bit two is true if driver plays at fixed frequency.

#define USESTIMER 8   // Bit three is true, if driver uses timer.

#define SHARESTIMER 16 // Bit 4, timer can be shared (BETA!!!!!)

#define LOOPEND 32		// bit 5, support looped samples, and pending sounds (BETA!!!)

#define STEREOPAN 64	// Bit 6, supports stereo panning.

#define STEREOPLAY 128 // Bit 7, supports 8 bit PCM stereo playback.

#define AUDIORECORD 256 // Bit 8, supports audio recording!

typedef struct

{

char far *sound;     // address of audio data.

unsigned int sndlen; // Length of audio sample.

int far  *IsPlaying; // Address of play status flag.

int	   frequency;  // Playback frequency.

} SNDSTRUC;

int  far DigPlay(SNDSTRUC far *sndplay); // 688h -> Play 8 bit digitized sound.

int  far SoundStatus(void);  // 689h -> Report sound driver status.

void far MassageAudio(SNDSTRUC far *sndplay); // 68Ah -> Preformat 8 bit digitized sound.

void far DigPlay2(SNDSTRUC far *sndplay); // 68Bh -> Play preformatted data.

int  far AudioCapabilities(void); // 68Ch -> Report audio driver capabilities.

int  far DigPakIdentityString(char far *str); // 68Ch -> reports ID string of

// sound driver. returns string length.

int  far ReportSample(void); // 68Dh -> Report current sample address.

void far SetCallBackAddress(void far *proc); // 68Eh -> Set procedure callback address.

void far StopSound(void); // 68Fh -> Stop current sound from playing.

void far *far ReportCallbackAddress(void); // 691h -> report current callback address.

void far DigPlayLoop(SNDSTRUC far *sndplay);

int  far PostAudioPending(SNDSTRUC far *sndplay);

#define NOTPLAYING 0  // No sound is playing.

#define PLAYINGNOTPENDING 1 // Playing a sound, but no sound is pending.

#define PENDINGSOUND 2			// Playing, and a sound is pending.

int  far AudioPendingStatus(void);

#define FULLRIGHT 0

#define FULLLEFT 127

#define FRONTANDCENTER 64

int far SetStereoPan(int panvalue); // 0-127, 0 full right.

//			 64 full volume both.

//			127 full volume left.

#define PCM_8_MONO 0

#define PCM_8_STEREO 1

#define PCM_16_MONO 2

#define PCM_16_STEREO 3

int far SetPlayMode(int playmode);

// Return 0 if mode not available, 1 if mode set.

int far SetRecordMode(int mode); // Set audio recording mode.

int far *ReportPendingAddress(void); // Reports the far address of the pending

// flag.	If this memory location pointed to by this address is 1 that means

// a pending sound effect is still pending.  When this becomes zero, then your

// application software can post the next piece of audio to play.  It is

// preferable to use this semaphore to know when to post the next buffer

// rather than to use the AudioPendingStatus call.

void far SetTimerDivisorRate(int rate); // Set the 8253 timer divisor rate.

// If your program, has reprogrammed the 8253 timer to another rate, then

// you must be sure to tell DigPak what that reprogrammed rate it.  Be

// sure to set this BACK to zero when your program exits!!!

int far *ReportSemaphoreAddress(void); // Reports the far address of the DigPak

// semaphore.  If this semaphore is true, then DigPak is currently active

// and you shouldn't post any DigPak calls.  This is EXTREMELY important if

// you are trying to invoke DigPak functions via a hardware interrupt, where

// you could potentially have interrupted DigPak itself.

void far StopNextLoop(void); // If a digital sound sample is looping,

// this call will cause it to stop looping, the next time it has

// finished.

int far ReportVersionNumber(void); // Report the DigPak version number.

// Return code is times 100, meaning that version 3.1 would be returned

// as the decimal number 310.  This function wasn't supported prior to

// version 3.1 release, so it will be returned as 0, for versions prior

// to 3.1.

/* Support routines */

void far WaitSound(void); // Wait until sound playback completed.

int  far CheckIn(void);   // Is sound driver available? 0 no, 1 yes.

/******************************************************************************/

/* Warning, it you enable LOADABLE_DRIVERS you must prodivde memory allocation*/

/* functions, and access to DOSCALLS.OBJ.				      */

/******************************************************************************/

#if LOADABLE_DRIVERS

int  far LoadSoundDriver(char far *name);  // Load a sound driver by filename, 1 success 0 fail.

void far UnloadSoundDriver(void); // Unload a previously loaded sound driver.

#endif

DIGPAK v2.0 README

2-23-92

__________________

This README.DOC file documents the changes for DIGPAK v2.0.

1.  Addition of the .ABX, Extended ACOMP File Format.

2.  Improved support of all memory models.

3.  Addition of DIGPAK API Functions #9 & #10.

Other changes in the DIGPAK API Functions.

4.  Provision of automatic detection of hardware

settings in SBLASTER.COM.

5.  Discussion of DOSCALL support routines.

6.  Sound Board support in the DIGPAK API.

(Media Vision ProAudio Spectrum now supported!)

7.  DIGPAK Version Log.

____________________________________________

1)

THE .ABX, EXTENDED ACOMP FILE FORMAT

------------------------------------

The new, extended ACOMP file format was developed as a result of user

request for an ACOMP file format without a size limitation.  In essence,

the .ABX file format is a collection of .ABT frames with a defining header,

as follows:

The .ABX File Format Header

---------------------------

Bytes 0-1: int  TotalFrames;		Total number of ACOMP frames in file.

Bytes 2-5: long int TotalSize;	Total size of original source file.

Bytes 6-7: unsigned int bufsize;	Frame buffer size used to compress in.

Bytes 8-9: unsigned int freq;    	Playback frequency of audio file.

....   ABH HEADERS[TotalFrames] 	Array of headers detailing the .ABT

frames, where each header is as follows:

typedef struct

{

long int fileaddress; 	  // Address in file of this audio section.

unsigned int fsize;		  // compressed file size.

unsigned int usize;		  // uncompressed file size.

} ABH;

Following is the diagram of a .ABX file with 3 .ABT Frames.

| ABX header/ABH/ABH/ABH | ABT frame | ABT frame | ABT frame |

|___|___|________|	   |	       |

|___|____________________|	       |

|________________________________|

See PLAYFILE.C for an example of double buffering (simultaneous play and

fetch) of a .ABX file.

NEW ACOMP OPTIONS

-----------------

The ACOMP utility also has 2 new switches as follows:

/v 	 - Normalize volume of input data.

/b<size> - ACOMP compression buffer size in K bytes.

Valid range is 1 - 64.  Default is 64.

ACOMP will compress audio data in the ABX

file format in <size> frames.

______________________________________________

2)

IMPROVED SUPPORT OF ALL MEMORY MODELS

-------------------------------------

In DIGPAK v2.0 all routines have been rewritten using FAR pointers

where applicable.  As a result, there no longer are different .OBJs

required for different memory models.

See DIGPAK.DOC in \THEAUDIO\DIGSRC for more details.

_________________________________________________

3)

ADDITION OF DIGPAK API FUNCTIONS #9 & #10

-----------------------------------------

The follwing details the new DIGPAK API Functions.

See \THEAUDIO\DIGSRC\DIGPAK.DOC for complete details.

Function #9: SetAudioHardware - Set hardware parameters.

-----------------------------

FUNCTION #9 IS NOT 100% RELIABLE IN V2.0 OF DIGPAK.

SEE DISCUSSION OF AUTO-DETECTION IN THE FOLLOWING SECTION.

INPUT: AX = 690h

BX = IRQ if device needs one set.

CX = BASE I/O Address, if device needs one set.

DX = OTHER, some other possible information the hardware

might need.

OUTPUT: NONE.

Certain harware can be reconfigured to different IRQ and base

address settings.  This function call allows the application

programmer to overide these default settings.  The interpretation

of these parameters might change from driver to driver.  Currently

only the SBLASTER.COM (Creative Labs SoundBlaster) driver can be

reconfigured.

FUNCTION #10: ReportCallbackAddress

-----------------------------------

INPUT: AX = 691h

OUTPUT: AX:DX -> far pointer to current callback address.

BX -> original caller's DS register.

This function should probably never need to be used by your

application software.  It is provided because the MIDPAK,

MIDI driver, needs to revector hardware callbacks so that

it can handle hardware contention problems between digitized

sound playback and synthesized sound playback.

OTHER CHANGES TO THE DIGPAK API FUNCTIONS

-----------------------------------------

The following are changes and additions to the existing DIGPAK API.

FUNCTION #5 AudioCapabilities

-----------------------------

Bit 3 - ON  => Timer is used by the loaded driver.

OFF => Timer is not used by the loaded driver.

_______________________________________________

4)

PROVISION OF AUTO-DETECTION OF HW SETTINGS IN SBLASTER.COM

----------------------------------------------------------

DIGPAK driver SBLASTER.COM now includes functionality to automatically

detect the IRQ and I/O address set on the SoundBlaster sound board.

These settings are detected at driver load time, after which they are

utilized.

_______________________________________________

5)

DISCUSSION OF DOSCALLS SUPPORT ROUTINES

---------------------------------------

The DOSCALLS support routines are provided as an efficient set of

library routines for file, disk, screen, and memory management functions.

DOSCALLS is a linkable object module that provides many of the functions

available through the standard C libraries.  There are number of reasons

to use DOSCALLS.OBJ rather than C library function calls.

1. Portability:	These functions work the same regardless of what

C compiler you are using.

2. Size:	These functions are extremely small compared to

equivalent C library functions. The C library

functions are general while the DOSCALLS

function calls are very specific.  Accessing C

library function calls causes large amounts of

library code to get drug in that you probably

don't want.

3. TSR capability: If you program TSR's you will find that

dragging in C library functions will kill you

for a whole number of reasons.  Using

DOSCALLS.OBJ you have no DGROUP dependencies and

you can easily write single segment TSR's and

even write both COM modules and DOS device

drivers.

4. Assembly language:	If you don't program in C and need basic

DOS support functions, you can just use DOSCALLS

for this purpose.

5. Simplicity: DOSCALLS don't ignore the fact that you are programming

on an MS-DOS machine.	They in fact simply provide

C callable access to INT 21h calls.  Because of this

they are all easy to use.

DOSCALL FUNCTIONS

-----------------

File Functions

--------------

mfopen		Do a DOS file open.  Returns a DOS file handle.

mfclose		Close a file that was opened with mfopen.

mfpos	 	Report current file position of the specified file.

mfseek		Seek file to the specified position.

mfread		Read from file, into address, for length of size.

mfwrite		Write data to file, from address, for length of size.

mdelete		Delete a file by filename.

fload		Allocate memory and read in entire file.

floadpara	A special version of fload, that reads the file into

allocated memory, but forces it to a paragraph boundary.

CurrentPath	Get the full, current path including drive letter.

LoadPath	Get the drive and directory we loaded from.

FindFile	Find a file by specification.

ChangeDirectory	Change to the specified drive and directory.

Keyboard Functions

------------------

keystat		Report DOS key status.

getkey		DOS getkey function. Returns keypress pending.

Extended key codes returned by getkey:

--------------------------------------

TAB	STAB	HOME	F1	F6

ENTER 	LEFT	PGUP	F2	F7

ESCAPE	RIGHT	END	F3	F8

SPACE 	UP	DOWN	F4	F9

BKSPACE	DOWN		F5	F10

String Functions

----------------

farcop		String copy routine using far pointers.

farcat		String concatenate routine using far pointers.

farlen		Return length of string using far pointers.

farcompare	String compare using far pointers.

ucase		Upper case a string using far pointers.

Memory Functions

----------------

fmalloc		DOS memory allocation.  Works fine by itself but

conflicts with C compiler's far memory allocation.

ffree		Free dos allocated memory.

all of the printf library code, which is HUGE!

fmov		Memory move using far pointers.

Text Output Functions

---------------------

tprint		Text print specifying color.

tcolor		Change text background color.

TextCursor	Position text cursor.

Timer INT Functions

-------------------

GetTimerInterruptVector	  Reports the current far address of the

timer interrupt vector.

SetTimerInterruptVector	  Set the timer interupt vector to a new

address and specify a new interrupt rate.

See DOSCALL.H in the \THEAUDIO\DIGSRC directory for complete details.

_______________________________________________

6)

SOUND BOARD SUPPORT IN THE DIGPAK API

-------------------------------------

Sound Driver support of the DigPak API:

The following table details the level of API support provided by

the DigPak sound drivers.

DigPak API Function

1   2   3   4   5   6   7   8  DMA  HW Res.     Mfg drvr req'd

Driver|                                                      Massaging req'd

------------------------------------------------------------------------

IBMSND	 Y   Y   Y   Y   Y   N2. Y   Y   N   6-bit log   N   Y   IBMSND

IBMBAK	 Y   Y   Y   Y   Y   Y   Y   Y   N   6-bit log   N   Y   IBMBAK

CVXSND	 Y   Y   Y   Y   Y   Y   Y   Y   N   8-bit       N   N   CVXSND

VMSND	 Y   Y   Y   Y   Y   Y   Y   Y   N   8-bit       N   N   VMSND

SMSND	 Y   Y   Y   Y   Y   Y   Y   Y   N   8-bit       N   N   SMSND

SBLASTER Y   Y   Y   Y   Y   N3. Y   Y   Y   8-bit       N   N   SBLASTER

ADLIB	 Y   Y   Y   Y   Y   Y   Y   Y   N   6-bit log   N   Y   ADLIB

TANEXTX	 Y   Y   Y   Y   Y   Y   Y   Y   N   4-bit log   N   Y   TANEXTX

TANSLTL	 Y   Y   Y   Y   Y   N3. N   N   Y   8-bit       Y   N   TANSLTL

SOURCE	 Y   Y   Y   Y   Y   Y   Y   Y   N   8-bit       N   N   SOURCE

DIGISP	 Y   Y   Y   Y   Y   N2. N   N   N   8-bit       Y   Y   DIGISP

ECHOII	 Y   Y   Y   Y   Y   Y   N   Y   N   8-bit       N   Y   ECHOII

LANTSND	 Y   Y   Y   Y   Y   N   N   N   N   8-bit       Y   Y   LANTSND

PAUDIO 	 Y   Y   Y   Y   Y   N   Y   Y   Y   8-bit       Y   N   PAUDIO

Notes:

1.  All sound drivers operate in the background except for DIGISP and IBMSND.

2.  Foreground only drivers cannot support ReportSample.

3.  ReportSample will be supported for DMA drivers as an approximation good to

1/18 second in the next version.

4.  Manufacturer drivers are required for TANSLTL, DIGISP, LANTSND, & PAUDIO.

5.  All DigPak sound drivers play 8-bit raw, digitized sound data.  However,

not all sound boards play this 8-bit format.  All DigPak sound drivers

"massage" the data on-the-fly.  If the associated PAUSE and destruction

of the sound buffer is unacceptable, Function #3 MassageAudio (Preformat

audio data into output hardware format) and Function #4 DigPlay2 (Play

preformatted audio data) can be used together to pre-massage and play the

audio data.  See the User's Guide for more details.

6.  The DigPak sound drivers DO NOT play compressed sound files.  Your

application must use UC.ASM to uncompress the sound files prior to playing.

DigPak API Functions are repeated here for your convience:

----------------------------------------------------------

1 	DigPlay			Play an 8-bit digitized sound

2 	SoundStatus		Check current status of sound driver

3 	MassageAudio		Preformat audio data into ouput HW format

4 	DigPlay2		Play preformatted audio data

5 	AudioCapabilities	Report capabilities of HW device

6 	ReportSample		Report current playback address

7 	SetCallbackAddress	Set a sound completion callback address

8 	StopSound		Stop playing the current soun

9	SetAudioCapabilities	Set HW parameters

10	ReportCallbackAddress

_______________________________________________

7)

DIGPAK VERSION LOG

------------------

v1.00  - 10/19/91	First production version of the DigPak sound drivers.

v1.01  - 10/25/91	Update to fix: Sound drivers not servicing timer INT.

v1.02  - 10/29/91	Update to fix: Covox drivers for timer INT bug of v1.01.

Added LDIGPLAY.ASM & LDIGPLAY.OBJ for large memory model

v1.03  - 01/07/92	Updated ECHOII.COM, SOURCE.COM, & PAUDIO.COM.

v1.04  - 01/25/92	Updated all drivers for MidPak compatibility.

v2.00  - 02/20/92	New release of DIGPAK & DIGPAK PRO.

_______________________________________________

End of README.DOC

MIDPAK and DIGPAK are THE Solution for DOS Multimedia!

Thanks for your support!

THE Audio Solution