ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-1

CHAPTER 4  FUNCTION KEYS

CONTENTS

4.0  Introduction

4.1  Using This Chapter Of The Manual

4.2  Help Keys

4.3  Cursor Movement Keys

4.4  Reading Keys

4.5  Command Keys

4.6  System Keys

4.0  INTRODUCTION

In order to run application programs in any computer, the

machine requires a program that will provide a means to

communicate between its various parts.  This program is called

an Operating System.  For example, the keyboard allows

characters to be typed, and the video screen allows them to be

viewed or spoken.  The operating system connects, or in computer

jargon "interfaces", these two parts together with a small

routine called the Monitor.

Generally, a Screen Review program links to the Operating

System, allowing what is typed or printed to the video screen to

be sent to a special routine that examines it before sending it

to the synthesizer.  Since the Enable Reader 4.0 program must

link with the Operating System, it seemed that the three year

evolution of the project would eventually lead to the total

integration of the Enable Reader System and the MSDOS Operating

System.  This makes MSDOS the world's first programmable talking

Operating System.

The Enable Reader Speech System is actually two programs in

one.  The Full Screen Speech Review System is designed for

visually impaired and blind people.  With Full Screen Speech

Review, a visually impaired person has access to the wide

selection of commercial software available for MSDOS compatible

machines, such as the IBM PC and dozens of other machines.

The second program built into the Enable Reader 4.0 Speech

System is the Talking Tutorial Programmer's Aid.  The

professional programmer now has access to many of the speech

functions of the Enable Reader 4.0 Speech System for writing

talking tutorials for software.  This gives blind and visually

impaired people the best of both worlds.  One, they have access

to the Review Mode of the Enable Reader 4.0 Speech System for

running commercial software; and two, any program that is

written with the Talking Tutorial Programmer's Aid will run for

them.

ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-2

The Enable Reader program is now designed to be a "Programmable

Resident Operating System Extension".  What "programmable" means

is that it is actually a part of the Operating System, and can

be used by programmers to write software that speaks by

accessing functions that you use in the Speech Review System.

It is "resident" in that the entire program is loaded into

memory at one time, and does not use time-consuming reads from

the disk drives to obtain parts necessary for its operation.  It

is an "extension" in that it extends the MSDOS Operating System

with a set of speech routines that are accessed through the same

supervisor call interrupts used by MSDOS or through keyboard

enhancemant program such as NEWKEY or SUPERKEY.  When in the

Review  Mode (where data can be reviewed from the video screen),

the  Enable Reader 4.0 System is a stand-alone Operating System.

It may be easier to understand just how the Enable Reader

program is accessed and how it operates, if you consider it to

be a separate invisible Operating System that can be called from

the MSDOS Operating System or an applications program at any

time.

When running the Operating System or an applications program,

you can type a simple toggle (a key that switches between two

states of operation) to enter and exit the invisible talking

operating system -- the Enable Reader 4.0 Speech System.  The

video screen and memory are locked down for your security while

you review the data on the video screen.  This way, nothing can

be changed in memory or on the screen while reviewing.

The two modes of operation are the INPUT MODE and the READ MODE.

The Input Mode is the side of the program where the operating

system and application programs run.  The Read Mode is the

Speech Review System where there are five categories of

functions used for screen review.  The CONTROL and LEFT OR RIGHT

SHIFT  key combination will toggle you between the two modes.  You

must  hold down the  CONTROL  key first, and then type the  LEFT

OR RIGHT SHIFT  key.

Another method of using the Review Functions is to use the Silent

Macro Mode entry point.  Unlike the CONTROL SHIFT (HARDWARE

INTERRUPT) method, the Silent Macro Mode is an ASCII or typeable

character. Used with keyboard enhancement programs you can create

unlimited functions that will automatically enter the READ MODE

review the data and EXIT.  The key to type when setting up your

own macros is CONTROL RIGHT BRACKET.  When you need to review the

macro program prompts, use the hardware interrupt method of

CONTROL LEFT OR RIGHT SHIFT.  Having both of these access methods,

Enable Reader runs with more software than any other screen review

system without them.

ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-3

There are three key combinations that will enter and exit the

READ MODE.  They are:

Control Right-Shift,

Control Left-Shift,

Control Right-Bracket.  THIS IS A RIGHT BRACKET  ]

The Control Right Bracket  ^]   combination is compatible with

both Kurtzweil 3000 and Votan VPC 2000 speech recognition systems.

When entering either mode, the synthesizer will announce it.

For example, when entering the Read Mode the synthesizer will

say  READ DATA , or when entering the Input Mode it will say

INPUT DATA.

All of the function keys are accessible only in the Read Mode.

The keys are NOT "case dependent", which means they can be typed

as upper or lower case letters.  Each function will perform its

task, and then return to the Read Mode Monitor where you may

type another function.  You will remain in the Read Mode until

you type  CONTROL  and  LEFT OR RIGHT SHIFT  to exit to the Input

Mode.

Some of the keys affect both the Read Mode and the Input Mode.

For example, the AUTO key when toggled off will put the entire

Review System into "Spell Mode".  All of the Reading Keys, such

as WORD, LINE, or PAGE, will speak all text as single

characters.  When in the Input Mode, all text that is typed will

be echoed immediately as single characters.  All of the

functions and details of their operation will be described in

the following sections of this chapter.

KEYBOARD KEY ENUNCICATION

The Enable Reader Professional Speech System has identification of

the keyboard keys in the INPUT MODE. The speaking of the keyboard

keys are tied to the help levels and with only a few exceptions

will be spoken unless the HELP LEVEL is set to no help.

The Backspace key will speak its name and the last character

typed on the screen.  If pressed more that once it will only say

the word BACKSPACE.

The Carriage Return or ENTER KEY will not speak as it is not

necessary once the position of the key is known.  It was

eliminated by the beta testers due to the constant use and

annoying speech.

The function keys now speak their names as F1, F2, etc.

The Alternate letter keys speak.  When the ALT key and a letter

are type the word ALT and then the letter will be spoken.

ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-4

The Escape, Delete, and Tab keys speak their names.

Many of the other keys are toggles or are toggled on when the NUM

LOCK key is on.  When the following keys are typed they will

announce whether they are toggled on or off.

CAPS LOCK          INSERT        NUM LOCK          SCROLL LOCK

When the NUM LOCK key is toggled on the following keys on the ten

key pad will speak the following when typed.

KEYSTROKE           PHRASE SPOKEN

HOME                HOME

END                 END

PGUP                PAGE UP

PGDN                PAGE DOWN

LEFT ARROW          LEFT

RIGHT ARROW         RIGHT

UP ARROW            UP

DOWN ARROW          DOWN

4.1  USING THIS CHAPTER OF THE MANUAL

There are five sets of Home Based Function Keys described in this

section of the manual.  The Integration keys are so different they

are described in their own chapter.

The five sets of keys are:

1 HELP KEYS

2 CURSOR MOVEMENT KEYS

3 READING KEYS

4 COMMAND KEYS

5 SYSTEM KEYS

Each Set of Keys will be described at the beginning of that

respective Key Set.  In addition, there are appendices that

contain the word definitions of the Enable Reader 4.0

Punctuation Set, and the Letter-To-Word Translator List of

words.

The format of describing each key is divided into seven

sections, described below:

SECTION 1     KEYNAME

The name of the key is the one or two word descriptor of

its function.  For example, the WORD key will read a word,

or the LINE key will read a line.

ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-5

SECTION 2     KEYSTROKE

This is the actual key that must be pressed to initiate the

function you choose to execute.  If it is a Control Key

function, you must first press the  CONTROL  key, and while

holding it down, type the secondary key -- such as CONTROL A.

SECTION 3     KEYTYPE

There are four "types" of keys:

1 SINGLE LEVEL:  A key that executes its function

immediately, and then returns to the Read Mode

Monitor.

2 MULTI-LEVEL:   A key that requires user input; and

when completed, returns to the Read Mode

Monitor.

3 USER DEFINED:  A key that requires the user to first

run a function that will define the key for

operation.

4 TOGGLE:        A key that acts the same as an ON/OFF

switch, or if it has more than two levels, will

continue through the different levels back to

the original level.

SECTION 4     SUMMARY

The Summary of a key gives a brief description of the

function performed, while the Description of the key below

gives complete details of how to operate the Key Function.

SECTION 5     DESCRIPTION

This section attempts to give the reasoning behind the

Function Key, how it is integrated into the Read Mode

System, and what special features it may have when used in

combination with other keys.

SECTION 6     USAGE

The Usage section gives an example of how the function

could be used in combination with other keys, what it is

primarily used for, and if there are other methods of

accomplishing the same task.

ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-6

SECTION 7     CROSS INDEX

The Cross Index section defines those keys that may be

affected by the use of the defined key, or other

combinations that may be used with it.

There are many different ways to combine keys so they will

perform the same function.  The use of the Enable Reader System

to achieve a level of professional competence will require

experimentation and usage in your own computer environment.

The methods of reviewing are so various and dependent on your

applications that we can not give you a strict definition of how

and when to use specific setups with a specific applications

program.

There are literally hundreds of thousands of programs, and

millions of visually impaired and blind people worldwide.  What

this  section of the manual attempts to do is to give general

rules of thumb for running most software applications we have

encountered to date.  Hopefully, you will write the Enable Talking

Software Company and share your knowledge with us so we may pass

it along in a newsletter on national databases.

4.2  HELP KEYS

CATEGORY DESCRIPTION:

The HELP Functions were the first keys to be defined in the

original Enable Reader 1.0 Prototype Version.  During the course

of the past three years they have been refined and extended.

The thirteen  Help Keys are now an integrated part of the Read

Mode System, and when turned on or off, affect other functions

that are used with them.  The following Help Keys are written in

alphabetical order.

KEYNAME:  ATTRIBUTE IDENTIFICATION

KEYSTROKE:  CONTROL G AS IN GENIE

KEYTYPE:  TWO LEVEL TOGGLE

The ATTRIBUTE Key is a toggle, which means it works the same as a

two way switch.  Type CONTROL G once to TURN ON ATTRIBUTE

IDENTIFICATION.  If you type it again ATTRIBUTE IDENTIFICATION

WILL BE TURNED OFF.

SUMMARY:  TURNS ON OR OFF ATTRIBUTE OR COLOR IDENTIFICATION.

ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-7

DESCRIPTION:

The ATTRIBUTE function  will help users to run programs such

as LOTUS 123, DBASE III, WORD PERFECT,  and HYPERACCESS.  Many

programs require that you know that a field or command is

highlighted, in reverse video, underlined, color blue on a white

backgound, or combinations of foreground and background colors in

order to proceed with the program.  The ATTRIBUTE IDENTIFICATION

KEY, gives you that information for most of the possible ATTRIBUTE

combinations available in the monochrome mode and all 128

foreground/background color combinations in the color mode.

ATTRIBUTE IDENTIFCATION is toggled on and off by typing the

CONTROL G KEY.  It may be easier to remember this if you think Of

G for graphics.  The prompt spoken is "attributes or color on"

when the key is active and "attributes or color off" when it is

inactive.  The Attribute Identification function for color works

in the review mode and in the INPUT mode.

Many programs write the character and attribute colors separately

and this  can lead to erroneous identification in the INPUT mode.

It is best to use color identification in the screen review mode

until a sighted person can verify the colors are correct as you

type them.

The attribute bytes in the color graphics mode give the names of

the colors instead of the monochrome values.  The five available

attributes are Normal, Reverse Video, Underlined, Blinking, and

High Intensity. The following are the possible combinations and

the phrases spoken:

MONOCHROME MODE

ATTRIBUTES                    PHRASE SPOKEN

Normal                        No Phrase Spoken

Reverse Video                 Reverse

Blinking                      Blinking

High Intensity                High

Underlined                    Underlined

Reverse Video-Blinking        Reverse Blinking

Underlined-High Intensity     Underlined-High

Reverse Video-High            Reverse-High

Intensity

Blinking-Underlined           Blinking-Underlined

Blinking-Underlined-High      Blinking-Underlined-high

Intensity

COLOR MODE

NOTE: IN COLOR MODE THE FIRST COLOR SPOKEN IS THE FOREGROUND

COLOR AND THE SECOND IS THE BACKGROUND COLOR.

ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-8

16 FOREGROUND COLOR ATTRIBUTES   (SAME PHRASE IS SPOKEN)

BLACK         BLUE             GREEN            CYAN

RED           MAGENTA          BROWN            LIGHT GREY

DARK GREY     LIGHT BLUE       LIGHT GREEN      LIGHT CYAN

LIGHT RED     LIGHT MAGENTA    YELLOW           WHITE

8 BACKGROUND COLOR ATTRIBUTES    (SAME PHRASE IS SPOKEN)

BLACK         BLUE             GREEN            CYAN

RED           MAGENTA          BROWN            LIGHT GREY

USAGE:

The Attributes or Color key will automatically check to see

whether the program you are running is in monochrome or color.

Many programs overlay screens on top of each other that are

different colors with different colored characters and some word

processors will put characters in different colors depending on

whether they are to be printed as bold, underlined, etc.  Other

programs will require that you know some prompt is in reverse

video or high intensity to make a menu choice.  Hopefully, the

documentation of the program will indicate this.  Most

documentation is so poorly written that you may need a sighted

person to teach you how the program is running visually.

CROSS INDEX:

NONE

KEYNAME:  AUTO

KEYSTROKE:  LETTER A as in ALIEN

KEYTYPE:  TWO LEVEL TOGGLE

The AUTO Key is a toggle, which means it works the same as a two

way switch.  Press the key once to have Spell Mode, and again to

return it to Whole Word Mode.

SUMMARY:  TURNS ON OR OFF WHOLE WORD OR SPELL MODE

DESCRIPTION:

The AUTO toggle has two states of operation.  Each state will

announce when it is entered.  One, the AUTO ON state, or Whole

Word Mode, is set when the Enable Reader program is first

loaded.  The second is the AUTO OFF state, or Spell Mode, that

is entered when the  A  key is pressed.

ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-9

In the Whole Word Mode, all blocks of letters, numbers, and

punctuation between two spaces will be spoken as one word.  For

example, if the letters are the word  COMPUTER , the synthesizer

will say  COMPUTER .  If there is a quote mark immediately

before and after the word COMPUTER, the synthesizer will say

QUOTE COMPUTER QUOTE.

In the Spell Mode, all characters are spoken individually.  This

means that when a WORD contains letters, they will be spelled to

you:  COMPUTER  will be spoken  C O M P U T E R .  If there are

numbers or punctuation, they will be spoken as whole words.  For

example,  "COMPUTER"  will be spoken as QUOTE C O M P U T E R

QUOTE.  If the word contains a punctuation character, it will

always be spoken as a whole word, and will not be spelled to

you.

The setting of the Spell Mode will not only affect the Read

Mode, but will also configure the Input Mode to immediately

speak all typed characters, including numbers and punctuation.

This effect is called "echoing".

USAGE:

The Spell Mode is used to spell out hard-to-understand words or

groups of letters.  Once it is turned on, it affects all of the

Reading Keys.  With AUTO OFF, the WORD key will spell a word,

the LINE key will spell a line, et cetera.  If you are reading a

directory of files on a disk and can not understand a file name,

it may be useful to turn off the AUTO function, thereby putting

the Reading Keys into Spell Mode; then go back and review.  When

used with the Window Keys, you can define the filename column as

a window and have each line of the directory spelled.

It is also important to keep track of whether the Spell Mode key

is ON or OFF, as the prompts in an application program will be

spelled to you.  If you forget to turn off the Spell Mode and

type the PAGE or ENDPAGE function, it will spell an entire

screen, thereby hanging up the synthesizer and the computer for

minutes while the data is sent.  As a precaution, there are QUIT

functions built into PAGE and ENDPAGE that will let you escape

and stop the synthesizer speech.

CROSS INDEX:

All Keys in Section 4.4 Reading Keys.

Note:  Escape procedures in Page and Endpage functions.

ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-10

KEYNAME:  CURSOR AT

KEYSTROKE:  LETTER C as in CEDAR

KEYTYPE:  SINGLE LEVEL

SUMMARY:  SPEAKS READ MODE CURSOR POSITION BY LINE AND COLUMN

DESCRIPTION:

The CURSOR AT Key will give you an exact screen location by line

number and column number.  The screen is divided into 25 lines

by 80 columns.  Lines are read from the top to bottom of the

screen, and always start with line 0 (zero).  The bottom line is

line 24.  Columns are read from the left to the right, and start

with column 0 (zero).  The far right margin is column 79.

USAGE:

The CURSOR AT function is typically used to identify locations

of columns or command lines that are used consistently in a

program.  Once they are located, the JUMP key can be used to

jump to a line and read it.  Alternately, the WINDOW keys can be

defined to read a section of the screen in columns or

paragraphs.

CROSS INDEX:

Window Setup:  Section 4.4 Reading Keys.

Jump, Y, X:  Section 4.3 Cursor Movement Keys.

KEYNAME:  IDENTIFY

KEYSTROKE:  LETTER I as in INDIA

KEYTYPE:  MULTI-LEVEL

SUMMARY:  IDENTIFIES READ MODE FUNCTIONS WITH BRIEF DESCRIPTION

DESCRIPTION:

The IDENTIFY Key is designed to give you a short description of

each function key while you are in the Read Mode.  To use the

IDENTIFY Key, first type the letter  I , then any key on the

keyboard.  The key will speak its function, or say  NOT A

FUNCTION KEY.  The IDENTIFY key will only speak one key function

definition, and then return you to the Read Mode Monitor.  For

the full text of the definitions of each key, see the section on

IDENTIFY Key descriptions.

ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-11

USAGE:

The IDENTIFY function is used to learn the different functions,

or to review a function before using it so you do not change

screen position inadvertantly.

CROSS INDEX:

None.

KEYNAME:  FILTER PUNCTUATION

KEYSTROKE:  LETTER M as in MIKE

KEYTYPE:  MULTI-LEVEL

SUMMARY:  USER DEFINED PUNCTUATION CHARACTER FILTER

DESCRIPTION:

The FILTER PUNCTUATION function provides for selectively turning

off and on individual punctuation characters.  The function will

prompt you to enter the punctuation character, and then will

announce whether it is filtered on or off.  The filter is

affected by the PUNCTUATION LEVEL function below.

If you want to filter on or off a character, type  M  and wait

for the  ENTER PUNCTUATION  prompt.  Next type any punctuation

character and listen to whether it is filtered on or off.  The

function acts like a toggle, so that if you type the same

character again it will give you the opposite message.  If the

character is not a punctuation character, the message  NOT

PUNCTUATION  will be spoken, and you will return to the  ENTER

PUNCTUATION  prompt.  After the character is identified as on or

off, you will be prompted again to  ENTER PUNCTUATION .  As many

punctuation characters as is necessary to be filtered may be

entered after typing  M  to use the FILTER PUNCTUATION function.

To exit the function, just type the  RETURN  key.  The  END OF

FILTER  message will be spoken, and you will return to the Read

Mode Monitor.

USAGE:

The FILTER PUNCTUATION function is used whenever there are long

strings of punctuation characters used in application menus or

when accessing databases.  For sighted people, these strings of

equal signs or minuses are used to separate columns or rows of

data, and by filtering them off you will not have to listen to

80 equal signs between lines.

ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-12

This method of filtering was preferred over the counting method,

whereby the 80 characters  are counted and a total given.  Because

when you know that a program has lines that you do not want to hear,

there is no need  to be reminded of this again and again as you run

the same  program year in and year out.

CROSS INDEX:

Punctuation Level:  Section 4.2 Help Keys.

Enable Reader 4.0 Punctuation Set:  Appendix A.

KEYNAME:  PUNCTUATION LEVEL

KEYSTROKE:  LETTER N as in NATIVE

KEYTYPE:  FIVE LEVEL TOGGLE

SUMMARY:  SELECTS BLOCKS OF PUNCTUATION CHARACTERS TO FILTER

OUT, OR ALLOWS DIRECT OUTPUT OF ASCII PUNCTUATION

CODES TO THE SPEECH SYNTHESIZER.

DESCRIPTION:

To provide a means of moving from one computer to another, the

Home Based Keys were used.  In order to transfer between

synthesizers without having to learn a new punctuation set for

each machine, the Enable Reader System uses one set of

punctuation for all machines.  Some synthesizers require that

punctuation characters be used for sending commands to change

their parameters.  The punctuation must be turned off to allow

the specific character to be transmitted, instead of the word

that Enable Reader translates it into.  The  N  key has five

levels of punctuation. They are ALL, MOST, SOME, NONE, and

direct OUTPUT TO SYNTHESIZER.  Each time the  N  Key is typed,

it moves down one level until it reaches OUTPUT TO SYNTHESIZER,

where it returns to the top level of ALL PUNCTUATION.

The five levels of punctuation and their prompts are:

1 ALL:   All punctuation characters will be spoken.

PROMPT: ALL PUNCTUATION

2 MOST:  All punctuation except  [ \ ] ^ _ ` { | }  (Ascii

Characters # 92 - 127)  will be spoken.

PROMPT: MOST PUNCTUATION

3 SOME:  All punctuation except those filtered off by MOST

and , - . / : ; < = > ? @   (Ascii Characters # 44

- 127)  will be spoken.

PROMPT: SOME PUNCTUATION

4 NONE:  No punctuation will be spoken, and the FILTER

PUNCTUATION function is not applicable.

PROMPT: NO PUNCTUATION

ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-13

5 OUT:   All punctuation bypasses the Enable Reader System

Punctuation Set, and is sent as Ascii characters

to the speech synthesizer.

PROMPT: PUNCTUATION OUT

USAGE:

The PUNCTUATION LEVEL function is used to quickly filter out

blocks of punctuation characters that are found to be an

annoyance when reading files or accessing databases with a

modem.  The most used punctuation characters in menu strings can

be turned off by using the SOME PUNCTUATION setting, as it turns

off the minus, period, equal sign, question mark, and comma.

CROSS INDEX:

Filter Punctuation:  Section 4.2 Help Keys.

Enable Reader 4.0 Punctuation Set:  Appendix A.

KEYNAME:  HELP WORDS

KEYSTROKE:  LETTER O as in OPEN

KEYTYPE:  FOUR LEVEL TOGGLE

SUMMARY:  SELECTS LEVEL OF KEYNAMES AND PROMPTS TO BE SPOKEN BY

FUNCTION KEYS

DESCRIPTION:

All of the Reading Keys have words that describe their function,

and then will execute it.  The Multi-Level keys have descriptive

prompts that ask for input or give an error message.  The

Pointer Movement Keys will speak their Keyname and execute their

functions.  If they try to move off the screen, the  TOP OF

SCREEN  or  BOTTOM OF SCREEN  may be spoken.

Some of the function keys have descriptive words that are

primarily used when first beginning to use the program.  They

will speak the KEYWORD and then execute it.  For example, type

the STEP Key and it will speak  STEP , and then execute its

function.  If the HELP WORDS are off, it will not say  STEP ,

but only execute its function.

The HELP KEYS toggles will always speak their KEYNAMES when they

are typed.  The multi-level HELP keys can have their Keynames

and Prompts for keyboard input turned off.  The HELP Levels are

designed so that the functions can be used more efficiently and

not break your concentration.

ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-14

The four levels of HELP WORDS and their prompts are:

1 All:   All HELP WORDS, KEYNAMES, PROMPTS and RANGES are

spoken.

PROMPT: ALL HELP WORDS

2 MOST:  The KEYNAMES of READING KEYS, MULTI-LEVEL POINTER

MOVEMENT KEYS, and USER DEFINED SETUP KEYS are not

spoken.

PROMPT: MOST HELP WORDS

3 SOME:  The POINTER MOVEMENT KEYNAMES, such as UP and

DOWN, are not spoken.

PROMPT: SOME HELP WORDS

4 NONE:  Only the quit key speaks the word QUIT and Disk

access errors are spoken.

PROMPT: NO HELP WORDS

USAGE:

The reasoning behind having levels of prompts is that after you

learn to use the function keys to locate data and read the

screen, it is more efficient, and there is more of a continuous

flow, if the STEP Key only speaks a character and then moves

ahead one, instead of saying  STEP letter STEP letter STEP

letter.  By typing the HELP WORDS Key and toggling to  MOST HELP

WORDS , none of the Reading Keys' or Cursor Movement Keys'

keynames will be spoken.

Try using the different functions individually at every HELP

WORD level to learn what names and prompts are left on or turned

off at each level.

CROSS INDEX:

None.

KEYNAME:  UPCASE

KEYSTROKE:  LETTER T as in TEACUP

KEYTYPE:  TWO LEVEL TOGGLE

SUMMARY:  TURNS ON OR OFF UPPER CASE LETTER RECOGNITION

DESCRIPTION:

The UPCASE function examines every letter to determine if it is

upper or lower case.  If it is upper case, the word  UPCASE

will be spoken before each upper case letter.  When the first

letter of a word is capitalized, the word will be spoken as

UPCASE word.  For example, if the word  Computer  is

capitalized, it will be spoken as UPCASE COMPUTER.

ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-15

The UPCASE function will identify upper case letters in both the

Read Mode and Input Mode.  When used in combination with the

TRANSLATOR function, the UPCASE function will take precedence,

speak  UPCASE , then translate the letter.

USAGE:

When entering data or running an application program, it is

sometimes necessary to enter upper case (capital) letters.  The

beginning letter of every sentence is capitalized.

CROSS INDEX:

Page, Endpage:  Section 4.4 Reading Keys.

KEYNAME:  TRANSLATOR

KEYSTROKE:  LETTER Z as in ZEBRA

KEYTYPE:  TWO LEVEL TOGGLE

SUMMARY:  TURNS ON OR OFF LETTER-TO-WORD TRANSLATOR

DESCRIPTION:

The TRANSLATOR function is used when it is necessary to

distinguish between hard-to-hear letters.  It only translates

letters, since numbers and punctuation are always spoken as

whole words.  When the TRANSLATOR function is turned on, all the

READING KEYS will translate letters to words from a table of

words used for this function.  For example, the WORD function

will translate the word  DATE  as  DELTA ALIEN TEACUP ECHO .  If

you leave the TRANSLATOR function on, and use the PAGE function

to read the whole video screen, it could take minutes for the

screen to be sent to the synthesizer.  The PAGE function has an

escape key that allows you to exit the PAGE function and stop

the synthesizer speech by sending the QUIT function.  The

Translator function works in both the Read Mode and the Input

mode.

USAGE:

The TRANSLATOR function was developed by a blind administrator

at the University Of California, Berkeley campus when he was

having a difficult time first using a speech synthesizer, and

could not understand certain letters.  We chose not to support

the Ham Radio Standard Word Translation List, due to the fact

that many synthesizers will pronounce the same words

differently.  It took many hours of patiently trying many

translation words with different synthesizers before this set

was chosen.

CROSS INDEX:

Page, Endpage:  Section 4.4 Reading Keys.

Letter-To-Word Translator Table:  Appendix B.

ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-16

KEYNAME:  SPACE IDENTIFICATION

KEYSTROKE:  SLASH KEY

KEYTYPE:  TWO LEVEL TOGGLE

SUMMARY:  TURNS ON OR OFF SPACE CHARACTER IDENTIFICATION

DESCRIPTION:

The SPACE function will turn on or off the space character

identification.  When the SPACE function is on, all spaces will

be spoken when using the Reading Keys.  Since both the WORD and

WORDBACK keys ignore spaces by jumping to the first letter of

the word, they will not identify them.  As with the UPCASE

function, the use of the PAGE function with the SPACE

IDENTIFICATION on will hang up the speech synthesizer for many

minutes.  The PAGE function has an escape key that allows you to

exit the PAGE function and stop the synthesizer speech by

sending the QUIT function.  The SPACE IDENTIFICATION function

works in both the Read Mode and the Input Mode.

USAGE:

Space identification is necesary when absolute accuracy is

required, as when typing in commands where syntax is critical.

It is also used when combining or editing text, and the document

must be visually correct.

CROSS INDEX:

Page, Endpage:  Section 4.4 Reading Keys.

KEYNAME:  CONTROL IDENTIFICATION

KEYSTROKE:  CONTROL O as in OPEN

KEYTYPE:  TWO LEVEL TOGGLE

SUMMARY:  TURNS ON OR OFF CONTROL KEY IDENTIFICATION

DESCRIPTION:

The CONTROL KEY IDENTIFICATION function will identify when a

Control Key has been typed from the keyboard.  The MSDOS

Operating System will print most of the Control characters on

the screen, and with the CONTROL KEY IDENTIFICATION on, will

speak the key twice -- once when it is typed, and again when it

is printed on the screen.

ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-17

USAGE:

Control Key identification is necessary in learning to use

programs that use Control keys to execute functions.  The

Wordstar wordprocessing program is based on a Control Key

System.

CROSS INDEX:

Video:  Section 4.6 System Keys.

KEYNAME:  SPACETALK

KEYSTROKE:  CONTROL Y as in YELLOW

KEYTYPE:  TWO LEVEL TOGGLE

SUMMARY:  TURNS ON OR OFF FORCED SPEECH WHEN SPACE BAR IS PRESSED

DESCRIPTION:

The Spacetalk key is a toggle that turns on or off forced

speech when the space bar is type. This feature works well when

wordprocessing as it will speak each word as you type it.  This

key is defaulted ON in the original defaults when the program

first starts up.

USAGE:

Although the spacetalk key works well with word processors it also

can be used effectively in any program that requires data input.

This key is used in conjuction with the delay key so it will not

only speak when the space bar is pressed, but when you pause for a

pre-selected time or when when a cariage return is pressed at the

end of a paragraph.

CROSS INDEX:

Delay: Section 4.2 Help Keys

KEYNAME:  WANG TERMINAL MODE

WARNING:  DO NOT USE THIS KEY WHEN RUNNING THE DOS ALONE AS IT MAY

INTERFERE WITH INTERNAL KEYBOARD HANDLING!!!

KEYSTROKE:  CONTROL U as in UNITED

KEYTYPE:  TWO LEVEL TOGGLE

SUMMARY:  TURNS ON OR OFF KEYBOARD INTERRUPT BYPASS

ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-18

DESCRIPTION:

The Wang Terminal Mode key is somewhat misnamed.  Although it is

written to be used with DPZ software's WANG terminal emulation

program, it is really designed to work with any program that takes

control of the keyboard dynamically.  These programs typically

have sections of the program that runs as a shell, a protected

mode, or emulators that use separate keyboard routines for set up

modes, etc.  When they take control of the keyboard they may cut

off the keyboard routine from passing characters to the screen

review program or even worse they can pass all the review

characters typed to a mainframe.  The Wang Terminal Mode key

eliminates this in most programs of this type.  Since there is no

way to know how a programmer wrote their keyboard interface

routines, there are no rules, but only exceptions when to use this

key.

CROSS INDEX:

Type; Video:  Section 4.6 System Keys.

END OF   SECTION 4.2  HELP KEYS

####