DUMPEM

DUMP Expanded/Extended Memory

User's Guide

INTRODUCTION

DUMPEM is a utility which allows you to display and edit

conventional, expanded and extended memory.  For example, you

can display the contents of an extended memory RAM disk, or

edit a page of expanded memory, or display the contents of the

system BIOS ROM.

The memory can be displayed in hexadecimal, unsigned decimal,

or decimal types, and in sizes of bytes, words, dwords, or

pointers.

DUMPEM also has built-in memory diagnostics tests which will

write patterns into the currently displayed block of memory,

and will highlight data which fails the test's verify logic.

The program was written primarily in P/LM-86, with a few

support routines written in ASM-86.

SYSTEM REQUIREMENTS

DUMPEM uses the BIOS INT 15h, functions 87h (block move) and

88h (return extended memory size) to manipulate extended

memory.  In addition, for ease of block transfer and editing,

the block move function is also used to display and edit

conventional and expanded memory.  Therefore, your system must

contain a BIOS (or device driver) which implements INT 15h,

functions 87h and 88h.

Machines which meet the above restriction are the IBM AT and

compatibles, IBM PS/2 Models 50, 60, 80 and compatibles, the

Inboard 386/AT, and the Inboard 386/PC.

The Inboard 386/PC is shipped with a device driver which, for

the first time, provides the INT 15h extended memory support

on a PC or XT machine, and makes use of the extended memory

hardware available on the Inboard 386/PC base board and

piggybacks..

INSTALLING DUMPEM

Simply copy the file DUMPEM.EXE from the diskette onto your

hard disk.

INVOKING DUMPEM

DUMPEM is invoked by typing:

DUMPEM

THE DUMPEM SCREEN

When invoked, DUMPEM will present you with a function key-

driven screen.

The screen is divided into three major areas.  The top-most

area lists the function key commands and associated status

information.  The bottom, open area displays the memory region

being examined/edited.  A third area is "popped" into the

screen center whenever additional status or error information

is displayed.

AN EXPLANATION OF THE FUNCTION KEYS and OTHER OPTIONS

ESC, X, x

DUMPEM can be exited by pressing ESC or 'X' or 'x'.

?

Pressing '?' displays a terse help screen which describes the

purpose of any keys which aren't listed on the main screen.

Ctrl-PgUp, Ctrl-PgDn

Pressing Ctrl-PgUp will display the previous 256 byte block of

memory.  Pressing Ctrl-PgDn will display the next 256 byte

block of memory.  Edit the memory start address (F9) if you

wish to "jump" to a specific address rather than "page" to it.

Left arrow, right arrow, up arrow, down arrow, PgUp, PgDn,

Home, End

Use these keys to move about the region you're currently

editing (either the memory data, page map array, or memory

start address).

When editing the memory data, you can insert/replace any digit

by typing the ASCII digits 0 - 9, and letters A - F (A - F if

the data is being displayed in hexadecimal type).  When

editing the memory start address, you can insert/replace any

digit by typing the ASCII digits 0 -9 and letters A - F.  When

editing the page map array, you can insert/replace any digit

by typing the ASCII digits 0 - 9.

Ins

Ins toggles the edit mode from insert (small cursor) to

replace (large cursor).

Rubout, Del, Space

Rubout deletes characters to the left of the cursor, and Del

deletes characters from the right of the cursor.  Space will

insert/replace digits with a space.  If an entire field is

overwritten with spaces, the field's value will be considered

0.

Enter (Carriage Return)

Enter will cause DUMPEM to accept the edit changes you've made

to the currently selected editable screen area (either the

memory data area, the memory address area, or the page map

array area).

<, >

< left-justifies all displayed data, and > right-justifies all

displayed data.

F1

F1 toggles through the four display size options: BYTE, WORD,

DWORD, and POINTER.

F2

F2 toggles through the three display type options:

HEXADECIMAL, UNSIGNED DECIMAL, and SIGNED DECIMAL.

NOTE: when POINTER size is used, only the HEXADECIMAL type is

available.

F3

F3 toggles through the twelve available diagnostics patterns:

FFFFFFFF, 00000000, 0F0F0F0F, F0F0F0F0, AAAAAAAA, 55555555,

A5A5A5A5, 5A5A5A5A, 01234567, 76543210, 89ABCDEF and FEDCBA98.

Once the pattern has been selected, you can subsequently press

F4 or F5 to automatically write the pattern to the selected

memory block and verify the results.

F4

F4 writes the specified pattern into the currently selected

block of memory (256 bytes in length).  After writing the

pattern into memory, DUMPEM verifies the write and will

highlight the data which doesn't match the expected pattern.

You will be prompted with a confirmation message asking you

whether you really want to write/verify the pattern.

NOTE: the verify logic is size-sensitive -- that is, if you

were displaying the data in WORD size, then the verify logic

will compare each WORD in the memory region with the low-order

WORD of the specified pattern.  If you were displaying the

data in DWORD size, then the verify logic will compare each

DWORD in the memory region with the entire DWORD of the

specified pattern.

F5

F5 writes the specified pattern from the currently selected

block of memory all the way up to the top of extended memory.

Each block of memory is written/verified with each of the

twelve diagnostics patterns.  You will be prompted with a

confirmation message asking you whether you really want to

write/verify the patterns.

NOTE: take great care when using functions F4 and F5.  They

overwrite the currently selected block of memory with the

specified pattern.

F6

F6 displays a summary of the system memory complement

(conventional, expanded, extended), displays the expanded

memory page frame address, and also tells you how many

expanded memory pages have been allocated to DUMPEM.

F7

F7 toggles through the list of active expanded memory handles.

Resident TSR or device driver software (like expanded memory

RAMdisks or print buffers) will typically have at least one

active expanded memory handle.

DUMPEM will allocate the rest of the available expanded memory

pages.  Use F6 to see how many expanded memory pages have been

allocated to DUMPEM.

Once you've toggled to the appropriate expanded memory handle,

you can press F8 to begin editing the physical-to-logical page

map array so that any of the handle's logical pages can be

mapped into the expanded memory page frame.  The page frame

address can be obtained via F6.

F8

F8 moves the cursor into the physical-to-logical page map

array.  Once there, you can begin editing the array, assigning

any logical page to any physical page for the selected

expanded memory handle.  Press Enter when you've finished

editing the array -- DUMPEM will then map in the pages.

F9

F9 moves the cursor to the memory address field so that you

can begin editing the memory start address.  The start address

can be on any boundary.  Press Enter when you want DUMPEM to

accept the edit changes.

F10

F10 moves the cursor to the memory data area of the screen so

that you can begin editing the memory data.  Press Enter when

you want DUMPEM to accept the edit changes.

LIMITATIONS AND DISCLAIMERS

DUMPEM is not an "official" product.  It has only been tested

by its author.

YOU are responsible for ensuring the memory data which you

edit directly, or overwrite with the test patterns, is

expendable.  The author is not liable for your actions.

GUARANTEES

There are none.