Notes for FIXMEMJR.SYS and RAMDRIVE.SYS

The following is a brief description of FIXMEMJR.SYS and RAMDRIVE.SYS.

This information will guide you in the installation and use of these programs.

Both of these programs are written in DEVICE DRIVER format and can only

be installed through the use of a CONFIG.SYS file.  The CONFIG.SYS file is a

text file similar to the BATCH files used by DOS to execute several commands

and/or programs by simply typing the name of the batch file.  A description of

the CONFIG.SYS file is covered in the DOS manual.  This document assumes that

you have a working knowledge of the CONFIG.SYS file.

The FIXMEMJR.SYS driver is used to re-arrange the memory map of the

PCjr.  The re-arranging will allow DOS and other applications to recognize and

use any available memory above the 128KB boundary.  The driver is installed by

inserting the following line in the CONFIG.SYS:

DEVICE=FIXMEMJR.SYS

One note to remember, the FIXMEMJR.SYS driver should not be installed if

any other memory recognition / relocation program is installed as this will

reduce the amount of available memory and give you nothing in return.

There are also 3 command line switches that can be added to the above

line that allow you to specify the amount of the video ram to set aside. These

options are defined as:

DEVICE=FIXMEMJR.SYS /GX

where X is a number between 1 and 3.  The number is referenced in the following

table.

1 - 32KB video buffer

2 - 64KB video buffer

3 - First 128KB reserved for DOS and video.

Each of these options has advantages and disadvantages.  Obviously the

more video ram you set aside, the less program ram you have available.	When no

command line switch is specified, the video buffer size is 16KB.  This is

sufficient for word processors and similar applications packages.  The 32KB

buffer will allow the use of the extended graphics screens, ie 320x200 16 color,

and 640x200 4 color.  Option 3 will increase the speed of some programs as long

as the memory above the 128KB is not as slow as the internal 128KB.

Also, since cartridge BASIC does not operate too well (graphics modes)

when the video memory has been relocated, I installed a trap to prevent

cartridge BASIC from being accessed when FIXMEMJR.SYS is installed.

#

The RAMDRIVE.SYS driver creates a RAMDISK that can be 10KB to 512KB in

size.  This driver must also be installed via the CONFIG.SYS file.  The command

line format is:

DEVICE=RAMDRIVE.SYS /size

One unique feature available with this program is that if no video

relocation programs such as FIXMEMJR.SYS are installed, the ramdisk resides in

the area above the 128KB boundary.  The drawback to this feature is that if the

video relocation program is installed after the ramdisk, the system will not

operate properly.  So be careful about this one.  I say this because some video

relocation programs can be installed at the DOS command level.	If you currently

use this type of program I strongly recommend you do not use the RAMDRIVE.SYS

device driver.

--Jeff McKee