If you notice any of the symptoms listed below, your drive

may be due for a little cleaning.

* Intermittent failures from a cold start or when in a cold

environment.

* PCjr hangs on boot (start-up), with the drive light lit

and typical drive

noise.

* Does not boot DOS and the drive may or may not fail

diagnostic test.

* Drive formats diskettes single sided instead of double sided.

(This usually indicates the read/write heads need cleaning.)

* Drive is noisier than you remember it to be when new.

* Many "sector not found" error messages.

* Weird error messages like "Missing/Dev/Con", "Bad or Missing

Command Interpreter", "Disk Bootstrap", or "Disk Boot failure".

* ERROR H may sometimes be caused by a "stuck" head due to dirty

rails.

CLEANING THE GUIDE RODS OF THE READ/WRITE HEAD CARRIAGE

Supplies:

Get 10 good firm swabs.  Cotton swabs are preferred

over foam swabs.  (the foam sheds particles)  A Q-tip swab

will work, but then you must pick out the fibers as they are

very loosely wound to the 'stick'.

Cleaning agents:

You have many choices of cleaners, but be safe by only using

isopropyl alcohol.  Normal flammability precautions apply.

Cleaning Procedure:

1) Remove the cover of the PCjr so you can see what you are doing.

2) Generously soak your swab in the cleaning agent (Alcohol),

so that it is ready to drip.

3) Wipe the swab on the rails that the Head/carriage asm

moves along.  (Use care to keep the swab away from the heads,

as heads don't like yukky stuff on them).

4) Throw away the used swab, and start with a fresh clean swab.

Follow the above steps until you are satisfied that both of

the guide rods are clean.

5) Allow the alcohol to run into the gap between the rail and

head carriage.  Then (be gentle of the Read/Write heads)

run the head carriage assembly back and forth by hand in an

attempt to flush the yukky stuff out and onto the rails where

it can be cleaned off. NOTE:  when sliding the head/carriage

along the guide rods, it should feel "raspy" or "jerky".

This is a normal condition caused by the stepping action of

the stepper motor that moves (steps) the head/carriage to the

next track on the diskette.

6) Replace the cover, and clean up your mess.

If repeated cleaning of the rails helps, but does not eliminate

your drive problem, you may need to perform the following

"tough rail"

cleaning.

1) Apply a small screwdriver to the top of each bearing (two-nylon)

with moderate downward pressure.

2) Work the head/carriage assembly back and forth several times.

3) Follow normal cleaning instructions 2 through 6 above.

The "tough rail" cleaning apparently serves to dislodge trapped

debris, or burnish the nylon bushing and railing to overcome a

burr.  No additional attention is warranted for the hidden short

right rail, as it is only a follower and not a bearing.

Last words of caution:

THE HEAD CARRIAGE HAS SELF LUBRICATING BEARINGS IN IT.

DO NOT USE OIL ON THE GUIDE RODS !!!

You can tell if your drive has been factory lubed.  Look at the

rods that the head slides along, and look at the carriage that

slides on the rods.  If you see what appears to be a very thin

grease, then more than likely the rails were lubed by the

factory.  If you can't see anything, then the rods were

probably not lubed by the factory.  If you find "dust balls"

on the head carriage or rods...please take the time to clean

the rods.  Your drive will appreciate it.

Conventional OILS AND GREASE COLLECT DUST.  BOTH WILL DRY OUT AND

BECOME GOOEY WITH TIME.  The use of thin oil may dissolve gooey

residues and sometimes clean rather than lubricate.  But the

lighter hydrocarbons in the thin oil that acted like a cleaner,

will eventually evaporate away again, leaving you with the

original problem.  A clean surface with light oil may work

indefinitely, depending on the type of oil that is used.

We do not recommend using silicone lubricants, as the carrier (solvent)

in pure silicone MELTS plastic!

Lubricating the Guide rods with a high quality oil will

initially make a drive run smoother

and quieter.  Then dust, and aging take their toll, making the head

access time take longer and longer, until errors occur once again.

Special words of thought for those who may have OILED

the head carriage guide rods in their PCjr diskette drive:

Depending on the environment the drive is in, you may never

experience a problem with the oil.  Or, you may start to see

some problems appear as soon as 1 year after the oil was added.

I leave you with the decision on relubing or oiling the guide rails.

John Dickinson (JLDKNSON at TUCVM2) states that new QUME PCjr

drives come with "Factory Lubrication" on the rails, so he guesses

that it is "factory approved".  John also notes that he too has

never seen another manufacturer recommend oiling the guide rods.

QUME drive owners have been reporting that their drive is usually

2 years old or older when it finally requires cleaning of the rails.

John Dickinson believes that QUME drives will need cleaning

every year or so after oiling the guide rods.

(Note that the "factory lube" life is projected to be 5 years

in an office environment.)  I have not experienced problems

with my factory lubed PCjr QUME drives.

Summary: Clean the rails per this procedure, and if your drive still

fails, clean the read/write heads per the RWCLEAN DOC.

Please send a reply to SLASH at RCHVMP3 should you find this

procedure to be inaccurate.

GOOD LUCK..................... Mike Shirk/Heath, PE.  RCHVMP3(SLASH)

Rochester MN           tie 553-7110

Document History:

V0.7      2-15-89    Added comments from Hank Kennedy

V0.6      2-02-89    Added aftermarket lubrication source

V0.5      5-11-88    Added comments from George Rudgers

V0.4     12-20-87    Added information on symptoms fixed by cleaning

V0.3      6-25-87    Added information on factory lube

V0.2      4-07-87    Added comments on identifying factory lube

V0.1     12-15-86    Added comments from John Dickinson

V0.0     12-15-86    Released document