SK-UPPS Diagnostics

October 1994

Software Version v1.13

1 Introduction

UPPSDIAG.EXE shows internal information about the UPPS DLI driver.

The versions of the drivers and the different settings can be viewed.

Before starting UPPSDIAG you have to load the Data Link Interface driver

(UPPSDLI.COM, UPPSJET.COM, UPPSISDN.COM, UPPSTR.COM, UPPSFI.EXE, UPPSFE.EXE

or UPPSFM.EXE).

UPPSDIAG -d shows all informations on one screen and exits.

Under WINDOWS and the DOS Task Switcher (DOSSHELL) you cannot connect to a

remote SNMPD.

You can leave the program after being asked back with ALT+F10 or without

being asked back with ALT+X.

2 The Menu

This chapter explains the values that are presented in the different menus.

2.1 Choose DLI

Choose one of the local DLIs presented or choose connecting to a remote

Simple Network Management Protocol Daemon (SNMPD).

2.2 Driver Status

- see online manual -

2.3 Network Statistics

The statistics window is split into two logical parts:

- The upper part contains statistics that are independant of the special

topology.

- The lower part contains topology specific statistics.

2.3.1 General statistics

RX demuxed

Number of packets/bytes received that were routed to protocols.

TX total

Number of packets/bytes sent by protocols.

RX multicast

Number of multicast packets/bytes received that were routed to protocols.

RX discarded

Number of received packets that were not routed to a protocol.

Uptime

The time the DLI is loaded (mod 24h).

CRC error

Number of received packets that had an illegal checksum.

Possible reasons for such packets:

- defective network adapter

- high frequency interference onto the network cable

- short-circuit in the network cable

Not byte bound

Number of received packets that have a number of bits that does not end

on byte boundaries.

Reasons as above.

Too long (rec.)

Number of received packets exceeding the maximum length for this topology.

This implies incorrect protocol drivers (on the remote station).

2.3.2 Ethernet specific

Misaligned pkts

A count of frames received on a particular interface that are not an

integral number of octets in length and do not pass the FCS check.

The count represented by an instance of this object is incremented when

the alignmentError status is returned by the MAC service to the LLC

(or other MAC user).  Received frames for which multiple error conditions

obtain are counted exclusively according to the error status presented to

the LLC.

FCS errors

A count of frames received on a particular interface that are an integral

number of octets in length but do not pass the FCS check.

The count represented by an instance of this object is incremented when the

frameCheckError status is returned by the MAC service to the LLC (or other

MAC user).  Received frames for which multiple error conditions obtain are

counted exclusively according to the error status presented to the LLC.

Single colls

A count of successfully transmitted frames on a particular interface for

which transmission is inhibited by exactly one collision.

A frame that is counted by an instance of this object is also counted by

the corresponding instance of either the ifOutUcastPkts or ifOutNUcastPkts

object and is not counted by the corresponding instance of the

ethStatsMultipleCollisionFrames object.

Multiple colls

A count of successfully transmitted frames on a particular interface for

which transmission is inhibited by more than one collision.

A frame that is counted by an instance of this object is also counted by

the corresponding instance of either the ifOutUcastPkts or ifOutNUcastPkts

object and is not counted by the corresponding instance of the

ethStatsSingleCollisionFrames object.

Bad signal

A count of times that the SQE TEST ERROR message is generated by the PLS

sublayer for a particular interface.

Deferred trans

A count of frames for which the first transmission attempt on a particular

interface is delayed because the medium is busy.  The count represented by

an instance of this object does not include frames involved in collisions.

Late collisions

The number of times that a collision is detected on a particular interface

later than 512 bit-times into the transmission of a packet.

Five hundred and twelve bit-times corresponds to 51.2 microseconds on a

10 Mbit/s system.  A (late) collision included in a count represented by

an instance of this object is also considered as a (generic) collision for

purposes of other collision-related statistics.

Excessive colls

A count of frames for which transmission on a particular interface fails

due to excessive collisions.

Int trans errs

A count of frames for which transmission on a particular interface fails

due to an internal MAC sublayer transmit error.  A frame is only counted

by an instance of this object if it is not counted by the corresponding

instance of either the ethStatsLateCollisions object, the

ethStatsExcessiveCollisions object, the ethStatsCarrierSenseErrors object,

or the ethStatsExcessiveDeferrals object.  The precise meaning of the count

represented by an instance of this object is implementation-specific.  In

particular, an instance of this object may represent a count of

transmission errors on a particular interface that are not otherwise

counted.

Carrier sense

The number of times that the carrier sense condition was lost or never

asserted when attempting to transmit a frame on a particular interface.

The count represented by an instance of this object is incremented at

most once per transmission attempt, even if the carrier sense condition

fluctuates during a transmission attempt.

Excessive defer

A count of frames for which transmission on a particular interface is

deferred for an excessive period of time.

Too long (rec)

A count of frames received on a particular interface that exceed the

maximum permitted frame size.

The count represented by an instance of this object is incremented when

the frameTooLong status is returned by the MAC service to the LLC (or

other MAC user).  Received frames for which multiple error conditions

obtain are counted exclusively according to the error status presented

to the LLC.

Length errors

A count of frames received on a particular interface with a length field

value that falls between the minimum unpadded LLC data size and the

maximum allowed LLC data size inclusive and that does not match the

number of LLC data octets received.

The count represented by an instance of this object also includes frames

for which the length field value is less than the minimum unpadded LLC

data size.

Big len field

A count of frames received on a particular interface for which the length

field value exceeds the maximum allowed LLC data size.

The count represented by an instance of this object is not incremented in

implementations that observe Ethernet encapsulation conventions (by which

the IEEE 802.3 length field is interpreted as the Ethernet Type field).

Int rec errors

A count of frames for which reception on a particular interface fails due

to an internal MAC sublayer receive error.  A frame is only counted by an

instance of this object if it is not counted by the corresponding instance

of either the ethStatsFrameTooLongs object, the ethStatsAlignmentErrors

object, the ethStatsFCSErrors object, the ethStatsInRangeLengthErrors

object, or the ethStatsOutOfRangeLengthFields object.

The precise meaning of the count represented by an instance of this object

is implementation-specific.  In particular, an instance of this object may

represent a count of receive errors on a particular interface that are not

otherwise counted.

2.3.3 FDDI specific

MAC upstream neighbour

The MAC's upstream neighbor's long individual MAC address.  It has an

initial value of the SMT-Unknown-MAC Address and is only modified as

specified by the Neighbor Information Frame protocol.

Note:	SMT-Unknown-MAC is 00:00:1F:00:00:00 (MSB) or

00-00-F8-00-00-00 (LSB, canonical).

MAC downstream neighbour

The MAC's downstream neighbor's long individual MAC address.  It has an

initial value of the SMT-Unknown-MAC Address and is only modified as

specified by the Neighbor Information Frame protocol.

Note:	SMT-Unknown-MAC is 00:00:1F:00:00:00 (MSB) or

00-00-F8-00-00-00 (LSB, canonical).

SMT version ID

The version used by this station for operation.  The value of this

attribute is 2 for this SMT revision.

SMT CF state

The CF_State represents the attachment configuration for the station or

concentrator.

CF-State ::= ENUMERATED {

isolated(0),

local_a(1),

local_b(2),

local_ab(3),

local_s(4),

wrap_a(5),

wrap_b(6),

wrap_ab(7),

wrap_s(8),

c_wrap_a(9),

c_wrap_b(10),

c_wrap_s(11),

thru(12) }

SMT frames send

The number of SMT frames sent.

SMT frames rec

The number of SMT frames received.

SMT ring up

The count of the number of times the ring has changed from the

'Ring_Not_Operational' state to the 'Ring_Operational'state.

This count is updated when a SM_MA_STATUS.Indication of a change in the

Ring_Operational status occurs.  Because of implementation considerations,

this count may be less than the actual RingOp_Ct.  It is not required that

this count is exact.

MAC frame cts

A count of the number of frames received by this MAC.

MAC error cts

A count of the number of error frames detected by this MAC that were not

detected by another MAC.

MAC lost cts

A count of the number of instances that this MAC detected a format error

during frame reception so that the frame was stripped.

Port x ler estimate

A long term average link error rate.  It ranges from 10^-4 to 10^-15 and

is reported as the absolute value of the base 10 logarithm.

Attach class

The capability of a station or concentrator for connection to an FDDI

network.  Stations and concentrators are classified as dual attachment,

single attachment or null attachment.

AttachmentClass ::= INTEGER {

single-attachment(1),

dual-attachment(2),

concentrator(3) }

Attach bypass

A flag indicating if the station has a bypass at its AB dual port.

AttachmentBypass ::= INTEGER {

true(1),

false(2) }

2.3.4 Token Ring specific

Upstream neighbour

The MAC's upstream neighbour's long individual MAC address.

Ring number

The unique number of the local Token Ring.

Ring up count

The number of times the station made sucessfully an 'open adapter'.

Signal losses

The number of times the adapter detected a loss of signal on the ring.

Lobe wire fault

The number of times the adapter detected an open or short circuit in the

cable between the adapter and the wiring concentrator.

Ring recoveries

The number of times the adapter observes claim token MAC frames on the

ring.  The adapter may be transmitting the claim token frames.

Line errors

The line error counter is incremented whenever

1) a frame is repeated or copied, and

2) the error detected indicator is zero in the incoming frame, and

3) at least one of the following conditions exists:

a) a code violation between the starting delimiter and the ending

delimiter of the frame,

b) a code violation in the token,

c) a Frame Check Sequence (FCS) error.

When the line error is incremented, the EDI of the frame is set to one so

that no further station counts the error.

Burst errors

The burst error counter is contained in all adapter configurations and is

incremented when the adapter detects the absence of transitions for five

half-bit times between SDEL and EDEL, or EDEL and SDEL.  Only one adapter

detects the burst five condition because the adapter that detects a burst

four condition (four half bit times without transition) directs its

transmitter to transmit idles if the burst five condition is detected.

ARI/FCI errors

The ARI/FCI error counter is incremented when an adapter receives an

Active Monitor Present (AMP) MAC frame with the ARI/FCI bits equal to

zero and a Standby Monitor Present (SMP) MAC frame with the ARI/FCI bits

equal to zero, or more than one SMP MAC frame with the ARI/FCI bits equal

to zero, without receiving an intervening AMP MAC frame.  This condition

indicates that the upstream neighbor is unable to set the ARI/FCI bits in

a frame that it has copied.

Lost frames

The lost frame error counter is incremented when an adapter is in transmit

(stripping) mode and fails to receive the end of the frame transmitted.

Receive congest

The receive congestion error counter is incremented when an adapter in the

repeat mode recognizes a frame addressed to it but has no buffer available

to copy the frame.

Uncopied frames

The frame copied error counter is incremented when an adapter in the

repeat/receive mode recognizes a frame addressed to its specific address

but finds the ARI bit not equal to zero.  This indicates a possible line

hit or duplicate address.

Token errors

The token error is active only in the active monitor station.  It is

incremented when the active monitor detects an error with the token

protocol as follows:

1) The MONITOR_COUNT bit of a token with nonzero priority equals

one.

2) The MONITOR_COUNT bit of a frame equals one.

3) No token or frame is received within a 10ms-window.

4) The starting delimiter/token sequence has a code violation in

an area where code violation must not exist.

DMA bus errors

The DMA bus error counter counts the occurrences of DMA bus errors that

do not exceed the abort thresholds as specified in the initialization

parameters.

DMA parity errs

The DMA parity error counter counts the occurrences of DMA parity errors

that do not exceed the abort thresholds as specified in the

initialization parameters.

Rec overflows

The Receive Overflow error counter counts the occurrences of packets which

do not fit in one DLI Mbuf (>4504 Bytes).

2.4 Protocols

Here you can view the protocols that are registered at the chosen DLI.

You can also view the receive mode of the protocol and for local DLIs the

multicast addresses the protocol registered.

2.5 Save Diagnostics

The information of the Driver Status, Network Statistics and Protocols

windows are saved to a file as ASCII text.