REV1100.DOC              MATROX ELECTRONIC SYSTEMS        IP-LIB Rev 1.1.0.1

+---------------------------------------------------------------------------+

|                         IP-8 Library for DOS                              |

|                                                                           |

|                          1303  Rev. 1.1.0.1                               |

|                                                                           |

|                              Nov. 18, 1991                                |

+---------------------------------------------------------------------------+

|                                                                           |

|      This document contains the following sections:                       |

|                                                                           |

|              1) Software Compatibility                                    |

|              2) Corrected Software Problems and Enhancements              |

|              3) Software Cautions                                         |

|              4) Hardware Cautions                                         |

|                                                                           |

+---------------------------------------------------------------------------+

+---------------------------------------------------------------------------+

|   1) Software Compatibility                                               |

+---------------------------------------------------------------------------+

BIOS Compatibility:

This new version of the IP-LIB Library will run with the BIOS Version

1.X.X.X and the next highest version.

IP8DRV Compatibility:

This new version of the IP-LIB Library will run with the IP8DRV.SYS

Driver Version 1.1.X.X and the next highest version.

The following memory models are available for Microsoft C Versions 5.1

and 6.0 in the corresponding files listed below.

MODELS     FILES

---------------------------

Small      MSC51\SIPLIB.LIB

Compact    MSC51\CIPLIB.LIB

Medium     MSC51\MIPLIB.LIB

Large      MSC51\LIPLIB.LIB

The following memory models are also available for Turbo C (from Borland)

Version 2.0 in the corresponding files listed below.

MODELS     FILES

---------------------------

Small      TCC20\SIPLIB.LIB

Compact    TCC20\CIPLIB.LIB

Medium     TCC20\MIPLIB.LIB

Large      TCC20\LIPLIB.LIB

For Intel 386 compiler the following library file is included.

MODELS     FILES

---------------------------

N/A	   IC386\IPLIB.LIB

REV1100.DOC              MATROX ELECTRONIC SYSTEMS        IP-LIB Rev 1.1.0.1

+---------------------------------------------------------------------------+

|   2) Corrected Software Problems and Enhancements                         |

+---------------------------------------------------------------------------+

****************** WARNING !!! WARNING !!! WARNING !!! *******************

All the library functions and variable names now begin with the VG_ prefix

to avoid duplication of names when you use IP-LIB with some other library.

Users of the lower IP-LIB version can continue to directly link programs

to this new library. A sub-include file (IPTRAN.H) can be included by the

main include files, IP.H and IPLIB.H, by specifying the compilation switch

"_IP_TRANSLATE" before including one of these main include files.

For example:   #define  _IP_TRANSLATE    /* Use the old IP_LIB name */

#include <ip.h>

This solution can be used temporarily, but we suggest that you change your

program to use the new VG_ name immediately. The IPTRAN.H file is provided

only in this release; it will not be supported in the next IP-LIB release.

***************************************************************************

. The VG_Read/VG_Write Block functions now work with buffers greater

than 64K.

. A new error code (-6) is now returned by the VG_InitLib() function. This

error code occurs when two programs try to use the board with IP-LIB. To

unlock the board, a program should call the VG_QuitLib() function. This

new error code has the following definition:

#define IP_BUSY  -6

With the new 1.1.0.1 Library this error code still be returned by the

VG_InitLib() function, but now the library can continue to work, and the

error code is meant to be a warning.

REV1100.DOC              MATROX ELECTRONIC SYSTEMS        IP-LIB Rev 1.1.0.1

Example of InitLib() call:

BYTE error;

/*

*  Don't forget to initialize the library and check if an error is found.

*/

switch ( error = VG_InitLib() )

{

/*

*   No Error or Warning found

*/

case IP_8:

break;

/*

*   This program can continue to run, but the setup of a previous

*   running program can be affected.

*/

case IP_BUSY:

printf( "WARNING: IP-8 Board already in used\n" );

break;

/*

*   When an Error is found the IP-8 Library function doesn't work.

*/

case IP_BOARD:

printf( "ERROR: IP-8 board or BIOS not present\n" );

printf( "       or wrong BIOS version\n");

exit(0);

case IP_DRIVER:

printf( "ERROR: Driver (IP8DRV.SYS) not present\n" );

printf( "       or wrong driver version\n" );

exit(0);

case MEM_POINTER:

printf( "ERROR: IP-8 memory Contention\n" );

printf( "       Couldn't create POINTERs to memory\n" );

exit(0);

default:

printf( "ERROR: IP-8 Library Initialization,\n" );

printf( "       Error code = %d\n", error );

exit(0);

}

+---------------------------------------------------------------------------+

|   3) Software Cautions                                                    |

+---------------------------------------------------------------------------+

. None for this release.

REV1100.DOC              MATROX ELECTRONIC SYSTEMS        IP-LIB Rev 1.1.0.1

+---------------------------------------------------------------------------+

|   4) Hardware Cautions                                                    |

+---------------------------------------------------------------------------+

. If WINDOW_PTR is used, the read/write to the IP-8 frame buffer,

the WORD access on the Access Buffer of the IP-8 should be made on the even

offset only.	You can bypass this problem by making a BYTE access on the

first ODD offset and continuing with some WORD accesses.

Notes: This problem is transparent, if you use the IP-8 library function.

. The grab state stays ON after a single grab, when GRAB_END_Y = V_BLANK_END

(offset 3F76 and 3F06 respectively).

. Don't try to make a grabback in the displayed frame with the zoom > 0.

. For the SetGrabSourcePos, you must have xsize >= 1 and ysize >= 2.

. To make a grabback, make sure to set:

SCAN_MODE = REPEAT_FIELD and GRAB_NI = 0

- or -

SCAN_MODE = PROGRESSIVE and GRAB_NI = 1,

- and -

select the EVENBOTH grab field.

Note: Default value for GRAB_NI = 0 (offset 3F65 bit 5).

. Artifact condition:

- Continuous grab ON.

- DISP_TYPE = MEMORY_VIDEO.

- Keyer output LIVE only or BOTH (LIVE and MEMORY).

- Interlace output OFF.

- Interlace input source.

- Vertical grab minification = 0.

- (GRAB_STR_Y - V_ACT_STR = DISP_POS_Y - GRAB_POS_Y) AND

(SCAN_MODE = REPEAT_FIELD)

- or -

(GRAB_STR_Y - V_ACT_STR = DISP_POS_Y - GRAB_POS_Y - 1) AND

(SCAN_MODE = PROGRESSIVE)

When you have all these conditions TRUE, you should have an inverse field

effect (every 4 pixels) when the image moves.

Put one of these conditions to FALSE to remove the artifact.

. You must use Cartesian window 0 for read only. The window 1 can be

accessed in read/write.

Note: Window increments (X and Y) must be always > 0.

. FIELD_ST (offset 3E5E bit 0) doesn't run properly in non-interlace.