Will work for PC/DOS 3.2 + (Maybe 3.1)

SET80 can be used to change the default drive parameters for the disk drives

on a PC/DOS machine. The default drive parameters are used by DOS formating a

disk. The rest of the time, the BPB (Bios Parameter Block) is read from the

first sector on the disk, and used to build the current drive parameters.

What use is it??

1) If you have a 3.5" drive configured as your B drive, and are running

PC/DOS 3.3 then you can use it to tell DOS that the drive has 80 tracks.

The alternative is to create a new logical drive using DRIVER.SYS. Earlier

versions allowed you to change the defaults by using the DRIVPARMS entry

in config.sys.

SET80 changes the entries for drive B so that it assumes 80 tracks on this

drive. This sets the B drive up for Quad density 5.25" drives, or 3.5 inch

drives. Both programs set up the formatting so that the disks are formatted

with a sector skew of 2. The normal sector skew for PC/DOS is 6. This

shouldn't cause any problems, but if you find that your IO is taking longer

than you expect, then just change the sector table in the program accordingly.

Documentation for the IOCTL call used may be found in the TECH REF Manual

3.1 (I believe.. certainly 3.2) or later. Forget either Norton, or the "NEW"

MSDOS book from Microsoft press.

If you have any questions give me a call....

Villy Madsen

MadHouse Systems Inc.

+1 403-929-8263

#########################################################################