Dmsound

Sound File Player for DeskMate

Version 0.7, May 16, 1996

by Jeffrey L. Hayes

CONTENTS

Description

Required Hardware/Software

Installation

Sound Devices

Sound Files

Main Screen

Mouse Interface

Keyboard Interface

Playing Headerless or Unrecognized Files

File Conversion

Backing Store

Setting the Volume

Saving the Configuration

For DeskMate Programmers

Acknowledgements

Other Programs (Blatant Plugs)

Contacting the Author

DESCRIPTION

Dmsound.pdm is a program to play sampled sound files under DeskMate,

Tandy's proprietary GUI, version 3.00 or later.  There is also a runtime

version available for DOS (see below).

Dmsound is still under development; I would appreciate comments and

suggestions.  This program is designed to be extensible to new file types

and devices, and interested parties are encouraged to participate in its

development.

This program is free.  Source code is available on request but requires the

DeskMate SDK for compilation.

REQUIRED HARDWARE/SOFTWARE

Dmsound requires either DOS 2.11 or later, or DeskMate 3.00 or later.  If

you have DeskMate 3, all the files needed to run Dmsound are included in

this archive.  Some DeskMate 3 programs will run under DeskMate 2,

particularly Professional DeskMate; I don't know whether this one will or

not.

If you don't have DeskMate, you will need the runtime version for Dmsound

as well as the files in this archive.  The file you need is DMSNDDOS.ZIP,

which should be on the same ftp site or BBS you got Dmsound from.  Read the

file README.DOS in that archive for instructions on installing the runtime.

If you can't find DMSNDDOS.ZIP, see the bottom of this file for a site.

If you have the DOS version of my program Dmgif (a GIF/JPEG viewer for

DeskMate), you can put Dmsound in Dmgif's directory, together with

DMSOUND.EXE from DMSNDDOS.ZIP (you don't need the other files in that

archive).  Dmgif and Dmsound use essentially the same runtime; the Dmgif

runtime has a few extra files that Dmsound doesn't need (see below).

NOTE:  Do NOT attempt to install or run the DOS version if you have

DeskMate!  You can wind up corrupting DeskMate so that you have to

reinstall to get it working again.  The runtime version can also cause

lockups on some Tandy systems running some versions of DeskMate.  Frankly,

if you have DeskMate 3, I don't know why you'd want the runtime version of

Dmsound anyway.

Dmsound requires a DeskMate-supported video adapter:  CGA, Hercules, EGA,

MCGA, VGA (or SVGA), Tandy 1000, Tandy 1000SL/TL/RL, or EGA with CGA

monitor (see README.DOS about the last one) - basically anything but MDA.

512k conventional RAM is probably the minimum (640k is better).  EMS or XMS

memory is helpful.  You should have either a hard drive (best) or 2 floppy

drives (not recommended).

Of course, you also need a supported soundcard (see below).  One such

"card" that everybody has is the PC speaker, but its use is not

recommended, simply because it sounds terrible (the PC speaker driver does

not work if Dmsound is run from inside Windows).

Dmsound will run fine on any Intel-compatible processor, 8088 or above,

provided the sound driver uses DMA.  A 286 or better is required to use the

interrupt-driven drivers (though this is not enforced).  A mouse (Microsoft

or PS/2) is supported, but Dmsound works fine without one.

INSTALLATION

To install Dmsound on a hard drive under DOS, first install the runtime

(see file README.DOS in DMSNDDOS.ZIP).  Then put the files in this archive

into the runtime directory.  You can then run Dmsound from the DOS prompt.

README.DOS has instructions on installing the runtime.

To run Dmsound under DOS on a system without a hard drive, just put all the

files in DMSNDDOS.ZIP and this archive on the same floppy.  The floppy must

*not* be write-protected, and it will have to be at least a 720k floppy.

See README.DOS for further instructions.  You can use a second floppy drive

for your sound files; the diskette with Dmsound on it must stay in the

drive at all times.  You will only be able to play short files with this

setup.

If you don't have DeskMate, but you do have a DOS runtime version of Dmgif

on your hard drive, put the files in this archive into the directory where

you have Dmgif installed.  Get DMSNDDOS.ZIP and extract the file

DMSOUND.EXE from it (you don't need the other files in DMSNDDOS.ZIP), and

put that in Dmgif's directory as well.  You will then be able to run

Dmsound from the DOS prompt (or from Dmgif).

There are two ways to install Dmsound on a hard drive under DeskMate - the

easy way, and the right way.  The easy way is just to put all the files in

this archive into the main DeskMate directory.  You can then run Dmsound

via the "Programs" listbox, or you can create a listbox for Dmsound using

the DeskTop F7 menu (use "*" for the extension).

The right way is to create a directory for Dmsound under the main DeskMate

directory and put the files in the new directory.  Then create a listbox

for it on your DeskTop giving the full pathname of DMSOUND.PDM for the

program, * for the extension, and the full pathname of the directory where

you keep your sound files as the startup directory.  One difference between

the easy way and the right way is that if I ever get around to creating an

online help file for Dmsound, you will only be able to use it under

DeskMate 3.02 or earlier if you have installed Dmsound in its own

directory.  Another difference is that Dmsound is easier to uninstall if it

is in its own directory.

If you have DeskMate on a floppy-only system, just put the files in this

archive onto a floppy.  Do not write-protect the floppy.  The PROGRAMS

listbox will display Dmsound's name when its floppy in in the drive (press

<control>-U when you change floppies).  You can use a second floppy drive

for your sound files.  As with a floppy-only DOS system, you will only be

able to play short files, and the diskette with Dmsound on it must stay in

the drive.

Dmsound will start up faster and take less disk space if you delete sound

drivers you don't need (see below).  *Don't* delete the PC speaker driver

(DMSNSPKR.RES), however; although it is essentially useless, it is

required.

SOUND DEVICES

This version supports the following sound drivers:

DMSNSPKR.RES    PC speaker (default but not recommended - 286+ required)

DMSN3VOI.RES    Tandy 1000/PCjr 3-voice sound chip (286+ required)

DMSNDAC.RES     Dumb DAC (as on a parallel port - 286+ required)

DMSNTDAC.RES    Tandy DAC

DMSNSB.RES      SoundBlaster 1.x

DMSNSB2.RES     SoundBlaster 2.x

DMSNPRO.RES     SoundBlaster Pro (8-bit stereo)

DMSN16MO.RES    SoundBlaster 16 (16-bit mono)

DMSNSB16.RES    SoundBlaster 16 (16-bit stereo)

The Tandy DAC is used on the Tandy 1000SL's, TL's, RL's, RLX, RSX, 2500's,

and Sensation model I, some Tandy laptops, and a few Digital and Panasonic

models.

The drivers for the PC speaker, 3-voice chip, and dumb DAC run off timer

interrupts, and only 286 or higher processors are generally fast enough to

do this, though the drivers have no 286 instructions to crash an 8086.

These drivers all work to some degree on a Tandy 1000TL (286) running at

4MHz ("MODE SLOW"), mostly for short files; they work acceptably for long

files at 8MHz.  The speaker driver won't work from inside Windows (and

won't work *well* in any case).

In addition, several file "devices" are supported for file conversion (see

"File Conversion," below).

To select an appropriate sound device, choose the "Select sound device..."

option in the "Defaults" menu.  A listbox will appear on the screen showing

available device drivers; if there are more than will fit, the listbox will

scroll.  With the keyboard, use the arrow keys or <page up> and <page down>

to highlight the driver you want, then press <enter>.  With a mouse,

double-click on the device name to select it, or click or press and hold on

the arrows to scroll the box.  Press <escape> or click on "CANCEL" to exit

the box without changing the current driver.

Generally, you won't be allowed to select a device you don't have, but

note:  some devices can't be reliably detected by software (such as the

dumb DAC).  If you select an undetectable device that you don't in fact

have, the results are unpredictable, but predictably bad.  If the driver

needs additional information about the device, a dialog box will be

displayed for you to enter the values (such as the I/O port number and DMA

channel).  The settings will be saved in a file, so you only have to enter

them once.

Notes on SoundBlaster-family drivers:  Dmsound won't use the BLASTER

variable; you have to tell it the base I/O port, IRQ number, and DMA

channel (have these handy the first time you run Dmsound).  If you get the

I/O port wrong, Dmsound will say "Device not present."  If you get the DMA

channel wrong, Dmsound will accept the settings, but when you go to play a

file, you won't hear any sound, and after about 10 seconds Dmsound will say

there was a "Hardware error on sound device."  If you get the IRQ number

wrong, the beginning of the file will play, but then it will stop, and

again you will get "Hardware error" after about 10 seconds.  If you get the

settings wrong, "Select sound device..." again and enter the right ones.

The current SoundBlaster 16 drivers do not support 8-bit DMA aliasing.

If you don't have working 16-bit DMA, use the Pro or 2.x driver.

Dmsound does not need and will not use Creative's DOS drivers, so you

can skip loading them.  They take up way too much conventional RAM anyway.

Device drivers are implemented as DeskMate resources.  Since they are

separate files, adding a new driver (when one is written) is as easy as

copying it to the directory where Dmsound resides.  Dmsound is designed

internally to accomodate 8- and 16-bit devices playing mono or stereo

sound, signed or unsigned PCM samples, at any sampling rate between 1000

and 65535 Hertz.  If you have a soundcard you would like to see supported,

and you are familiar with programming it in assembly language, contact the

author for assistance in writing a driver.  The driver definition is also

extensible to include new capabilities.

SOUND FILES

This version supports the following sound file types:  RIFF WAVE (.wav)

files in standard Microsoft PCM format (format code 1), Microsoft ADPCM

format (format code 2), CCITT A-law format (format code 6), and CCITT mu-

law format (format code 7); Tandy .snd files, compressed or uncompressed,

new or old format; Amiga .mod sample (.sam) files; Amiga IFF (.iff) files;

Apple or SGI AIFF (.aif) files; Apple AIFF-C (.afc) files (uncompressed

only); Sun or DEC .au files and NeXT .snd files, in 8-bit PCM, 16-bit PCM,

A-law, or mu-law format; STEVE files; Creative Labs .voc files, in 8-bit

PCM, 16-bit PCM, A-law, or mu-law format; and headerless sound files.  Mono

or stereo files can be played.

There is some support for Mac sound resource files in MacBinary FSSD

format, including support for HCOM compression.  Unless the file is

compressed, however, Dmsound can't determine the sampling rate of a Mac

file from its header.  The settings in the "Defaults" menu will be used in

this case.  Unfortunately, many Mac files are still treated as headerless

(see "Playing Headerless or Unrecognized Files," below).

The file type is determined by Dmsound based on the file's extension and

its internal data.  Hence, a RIFF WAVE file, for example, must have a .wav

extension to be recognized as such.  If the file is recognized as a type

that is not supported, such as a RIFF WAVE file not in a supported format

(Creative Labs ADPCM, for example, is not supported), a message is

displayed, and the file is not played.  If the file is not recognized at

all, it is treated as a headerless sound file (see "Playing Headerless or

Unrecognized Files," below).

The following file extensions are significant to Dmsound:

.wav         Microsoft RIFF WAVE (Windows 3.x & 95)

.snd         Tandy DeskMate .snd or NeXT .snd

.au          Sun or DEC .au (if headerless, assumed mu-law, 8012Hz)

.sam         Amiga .mod sample (headerless signed mono 8-bit 8363Hz)

.voc         Creative Labs .voc

.iff         Amiga/Electronic Arts 8SVC IFF (.aif, .afc also accepted)

.aif         Apple/SGI AIFF (.iff, .afc also accepted)

.afc         Apple AIFF-C (AIFF w/optional compression, .iff, aif OK)

Except for .sam and .au, Dmsound checks the file header and treats the file

as headerless if it doesn't match.

Sun ADPCM is recognized, but at this point you probably need a Pentium to

play in real time.  Hopefully, I'll get around to optimizing it some.

Dmsound has a modular internal structure, permitting new file types to be

added (relatively) easily.  Given sufficient information about the file

format, support for the new format can be added with a single C source

file.  If know about file format you would like supported, write to the

author.  Dmsound can convert internally any 8- or 16-bit, mono or stereo,

signed or unsigned sound data, at any rate between 1000 and 65535 Hertz, to

a format and rate usable by the soundcard.  New compressed formats can also

be accomodated, provided code is available to convert them to PCM.

MAIN SCREEN

Like other DeskMate programs, Dmsound's screen is divided into three parts:

the title line, the menubar, and the work area.

The stuff on the title line is standard for all DeskMate programs, though

the arrangement varies from version to version.  There is a "help" prompt,

which Dmsound doesn't use.  I haven't written a help file for Dmsound yet,

so if you press F1 you'll either get general DeskMate help or an error

message (F1 works in Setup, though).  The current date and time are also

displayed.  The time can get behind when playing sound, but it usually

catches up when the sound is over.  The name of the program, "Dmsound," is

also there, along with the "datafile."  The "datafile" for Dmsound is the

current sound directory.  It is abbreviated to fit in 28 characters or

less.

Below the title line is the menubar.  Here you have the available pulldown

menus and the function keys you press to invoke them.  The menubar, too, is

partly standardized.  F10 is the accessory menu.  This menu contains

various applets ("accessories" in DeskMate) that can run on top of the

application; if you have the DOS runtime version of Dmsound, the only

accessory you have is Setup - it is the only accessory Dmsound needs.  You

use the Setup accessory to select a mouse driver and change screen colors.

There are other settings in Setup, but they don't matter for Dmsound.  F9

is the alarm menu.  Many long-time users of DeskMate don't know what it is;

DeskMate comes with an "alarm clock" accessory that uses this menu to

inform the user that the alarm has gone off.  If you have the DOS version,

the F9 menu is completely useless and will always be empty.

The other menus are Dmsound's menus.  There are three of them:  "File",

"Defaults", and "Volume".  The "File" menu is the one you will use most

often.  There you will find items to:  type in a file to play; type in a

directory to change to; delete the highlighted file; rename the highlighted

file; copy the highlighted file; save the configuration (see "Saving the

Configuration," below); exit the program; run another DOS or DeskMate

program; update the list of files on the screen (for example, if you change

floppies); and display the version information for Dmsound.  If you select

"Run..." from the file menu in the DOS version of Dmsound, then when the

other program exits you will come back to Dmsound.  It's not exactly a

shell escape; rather, Dmsound exits, the other program runs, then Dmsound

restarts using the current configuration settings (see "Saving the

Configuration").

The "Defaults" menu contains some stuff you might want to set, but mostly

you will just set it once, "Save configuration" in the "File" menu, and

forget it.  The first four options have to do with "headerless" files -

files that Dmsound doesn't recognize as being some particular type of sound

file (see "Playing Headerless or Unrecognized Files," below).  The fifth

option, "Storage options...", pertains to how Dmsound gets extra storage

when conventional RAM is not enough; it is described in detail below (see

"Backing Store," below).  "Select sound device..." is what you use to pick

a soundcard (or file "device") and set up Dmsound for it (see "Sound

Devices," above).  "Enable looping" controls whether sound files that

contain repeat loops are actually allowed to repeat (some files are

designed to play forever).

The "Volume" menu contains only two items, "Set volume..." and "Amplify

samples".  These are described under "Setting the Volume," below.

Below the menubar is the work area.  Here is where Dmsound displays the

files in the current sound directory, along with subdirectories and drives.

Drives come first, then subdirectories, then files.  Drives and

subdirectories are displayed in bold.  There can be up to 525 drives,

files, and subdirectories in a directory; if there are more than fit on a

screen, a scrollbar will appear at the bottom of the screen.  One of the

entries in the directory list will always be highlighted in reverse video,

and menu options, mouse actions, and keystrokes operate on the highlighted

item.

MOUSE INTERFACE

To play a file in Dmsound with a mouse, double-click on it.  To move the

highlight to a file without playing it, click on it once.  To select a menu

item with the mouse, press and hold the mouse button on the menubar button

(the menu will be pulled down) and drag the mouse to the option you want.

If you see a pushbutton (such as "OK" or "CANCEL"), click on it to push it.

In the "Storage Options" box there are some checkboxes - click on them to

toggle the "X".  In the "Select Sound Device" box, there is a listbox.  You

can click on an arrow in the listbox to move the highlight, press and hold

on an arrow to scroll quickly, click once on an item to select, and double-

click to select and go (like selecting, then pressing "OK").  Clicking on

an editfield moves the focus to the editfield so that you can type in it.

Clicking on a radio button presses it.

If there are more drives, subdirectories, and files in the current sound

directory than will fit on one screen, a scrollbar will appear at the

bottom of the screen.  Click once on the arrow buttons to move the

highlight one column in either direction.  Click once between the elevator

(box) and an arrow button to move the highlight one screenful in either

direction.  Press and drag the elevator to move the highlight to a relative

position in the list.  Press and hold on an arrow button to move quickly in

the direction of the arrow.  Press and hold between the elevator and an

arrow button to move even more quickly.  Scrollbars in dialog boxes work

similarly.

Dmsound only uses one mouse button.  If you have a two-button mouse, the

buttons are treated the same.  If you have a three-button mouse, use either

the left or the right button; the middle button doesn't work.  DeskMate

supports the use of a joystick as a mouse, but Dmsound doesn't - the

joystick "mouse" interferes with sound playback.  If you select the

joystick "mouse," Dmsound will disable it (and tell you so).

KEYBOARD INTERFACE

To play a file with the keyboard, move the highlight to it and press

<enter> or <space>.  Many keys are significant to Dmsound in some way.

Here they are:

<enter> or <space> plays the highlighted file or changes to the highlighted

drive or directory.

Function keys F2, F3, F4, F9, and F10 pull down menus.  Use the <up arrow>

and <down arrow> keys to move between menu items, and press <enter> to

select a menu item.

<tab> and <shift>-<tab> move between the items in a dialog box.  Sometimes

arrows work too.  <enter> presses the pushbutton that has the focus, or if

the focus is not on a pushbutton, it presses the "OK" button.  <space> also

presses a button.  <space> toggles the "X" in a checkbox.  <escape> presses

the "CANCEL" button.

<up arrow> and <down arrow> move between items in a listbox.  <page up> and

<page down> move a boxful at a time.  <control>-<home> moves to the top of

the listbox, <control>-<end> goes to the end.

Arrow keys, <backspace>, <delete>, <home> and <end> all work in editfields.

Some editfields will scroll (those asking for file paths, for example).

In the file list, <left arrow> moves the highlight one column to the left.

<right arrow> moves it one column to the right.

<tab> or <down arrow> moves the highlight down to the next entry in the

file list.  If at the bottom of a column, it goes to the top of the next

column.  If at the end of the list, it goes back to the beginning.

<shift>-<tab> or <up arrow> moves the highlight up to the previous entry in

the file list.  If at the top of a column, it goes to the bottom of the

previous column.  If at the beginning of the list, it goes to the end.

<home> moves the highlight all the way to the left on the current row,

scrolling as necessary.  <end> goes all the way to the right.

<page up> moves the highlight to the top row in the current column.  <page

down> goes to the bottom row.

<control>-<home> moves the highlight to the first entry in the list

(normally drive A:).  <control>-<end> goes to the last entry in the list.

<control>-<page up> is the "page left" key.  It moves the highlight left

one screenful at a time.  <control>-<page down> is "page right":  it goes

right one screenful at a time.

Keys for printable characters, like letters, numbers, etc., move the

highlight to the next entry in the list that starts with that character.

If there are no more entries that start with the character, the highlight

moves to the first entry in the list that starts with the character.  If no

entry starts with the character, the key does nothing.

<control>-D or <delete> deletes the highlighted directory or file.

Directories must be empty to be deleted.  You will be asked for

confirmation.

<control>-R renames the highlighted directory or file.  You will be asked

for the new name.

<control>-C copies the highlighted file.  You will be asked for the path of

the new copy.  You can give either an absolute or relative path.

<control>-V sets the volume.  You will be asked to enter it as a percentage

of full volume (see "Setting the Volume," below).  If your soundcard does

not support setting the volume, the key does nothing.

<control>-U updates the screen, rereading the file list from disk.  It may

be useful if you are playing files on floppy disk.

<escape> exits the program.

There are a few more keys, but they are less useful.  They work mainly in

editfields.

PLAYING HEADERLESS OR UNRECOGNIZED FILES

To play a headerless (or unrecognized) sound file, Dmsound uses global

defaults you supply.  You must specify the sampling rate in Hertz for the

file, whether the file includes signed or unsigned samples, whether the

file is 8- or 16-bit, and whether the file is mono or stereo.  To change

these settings, use the "Assume sampling rate...", "Assume signed", "Assume

16-bit", and "Assume stereo" options in the "Defaults" menu.  Many file

types not directly supported by Dmsound can be played in this manner,

though you have to play with the settings some.

To play a file that Dmsound doesn't recognize, you proceed in steps, trying

different settings till the file sounds right.  First you have to figure

out whether the file is 8- or 16-bit, signed or unsigned; if you don't get

those right, the file will sound like rank noise.  Most 8-bit files are

unsigned, and most 16-bit files are signed.  When you determine the right

settings for these two, the file should sound like *something*, though it

may sound like chipmunks on speed or Lurch on a slow day.

If you tried all four possibilities and the file still sounds like rank

noise, it may be a headerless Sun mu-law file.  Try renaming it with the

.au extension and see if it plays then.  If not, either it isn't a sound

file, or it's compressed or encoded in some way.  It is common for sound

files to be distributed as compressed archives, such as .zip or .arc; get

Pkunzip or Arce respectively to dearchive the files.  If the file contains

readable text (try looking at it with a text editor or typing it to the

screen), it was encoded for transmission by email or posting on the

Internet.  The most common methods of encoding are uuencode (xxencode is a

variant used by IBM) and Mime.  Get DOS Uudecode for the former; some mail

programs can decode Mime.  Finally, the sound file may use compression

itself, usually some form of ADPCM.  Unfortunately, unless Dmsound has been

expressly written to handle those files, they can't be played.

If you have some kind of recognizable sound, the next thing you need to

figure out is whether the file is mono or stereo.  Obviously, it helps to

have a stereo soundcard to do this (SoundBlaster Pro and SoundBlaster 16

are stereo cards).  If you don't have a stereo card, just see whether

making it stereo sounds better than otherwise.  Most stereo files will

sound fine in mono if played back at double the rate they were recorded at.

The last step is to determine the sampling rate.  If the rate is too low,

the file will sound like Lurch; if too high, like the Chipmunks.  Common

sampling rates are 11025, 22050, 44100, and to a lesser degree 8000 and

5512.  If Dmsound won't allow you to set the sampling rate high enough to

make the file sound right, it's a stereo file.  Go back and change that and

try again.

The "Assume Sampling Rate" dialog box works like the "Set Volume" dialog

box (see "Setting the Volume," below).  There is an editfield on the top

left for typing in a sampling rate, and a scrollbar on the right for

setting it by mouse.  To use the keyboard method, tab to the radio buttons

on the top and press <space> on the button over the editfield.  To use the

mouse method, click on the radio button over the scrollbar.  If using the

keyboard, press <enter> after typing the rate to set it, <escape> to cancel

out of the box, or <tab> to see the rate on the scrollbar.  If using the

mouse, you can drag the scrollbar elevator to select the rate, click on an

arrow button to change the rate by 5 Hertz in that direction, press and

hold on an arrow button to change by 5 repeatedly, click between the

elevator and an arrow button to change by 250, or press and hold between

the elevator and an arrow button to change by 250 repeatedly.  The

editfield will track the rate during a mouse drag or hold.  The radio

button setting will be remembered, and it will be saved in DMSOUND.CFG when

you save the configuration (as will the default rate itself).

If you like the file and want to play it again, you should convert it to a

recognized file type so you won't have to go through this again.  See "File

Conversion," below.  The converted file will have a click at the beginning.

That is normal.  If you have the DeskMate Sound editor, you can convert to

Tandy .snd and use Sound to delete the click.

Most Mac files don't have headers than an IBM PC can use, so you have to go

through the procedure above to play them.  If the Mac file is an old one,

it is probably an 8-bit mono unsigned file.  Dmsound will recognize Mac

sound (FSSD) files with an .snd extension as 8-bit unsigned mono, but only

if they still have the MacBinary header attached; otherwise, they will be

considered headerless (many old Mac files do not have MacBinary headers).

If in addition a recognized Mac .snd file is compressed with HCOM

compression, Dmsound will be able to get the sampling rate from the header

too.  Otherwise, the "Assume Sampling Rate..." setting will be used to play

the file.  Old Mac files are usually 22050, 11025, or 5512Hz.  Note that

there are some newer FSSD files that are *not* 8-bit mono, and Dmsound will

misconstrue them as that if they have an .snd extension - you need to

rename the file in that case.

Newer Mac files don't have any particular extension (Macs use a

resource fork rather than an extension).  They can be either 8- or 16-bit,

mono or stereo, at any sampling rate.  Note that .sit files are archives,

not sound files.  You need to get Unstuffit for DOS to dearchive them.

Similarly, .hqx files are encoded.  You need DOS Binhex for them.

The settings for headerless files are saved in DMSOUND.CFG when you save

the configuration (see "Saving the Configuration," below).

FILE CONVERSION

Dmsound supports file type conversion using the following file "devices":

DMSNTSND.RES    Tandy .snd (old-format, uncompressed, 11kHz)

DMSNW8M.RES     8-bit mono .wav

DMSNW8S.RES     8-bit stereo .wav

DMSNW16M.RES    16-bit mono .wav

DMSNW16S.RES    16-bit stereo .wav

The .wav files are written in standard Microsoft PCM format, at the same

sampling rate as the input file.

To convert a file, use the "Select sound device..." option in the

"Defaults" menu to select one of the file "devices," then play the file.

The input file will be loaded and "played" as if you were listening to it,

but you won't hear any sound.  Instead, a file will be created on your

disk, containing the sound in the new file format.

Normally, a dialog box will be displayed asking you for the name of the

output file.  The default name is NEWFILE.???, where ??? is the usual

extension of the output file, for example, NEWFILE.SND or NEWFILE.WAV.  If

you accept the default, the output file will be written to the directory

where you installed Dmsound.  If that is not where you want the file to go,

specify a full pathname at the prompt.

If memory is extremely tight on your machine, the dialog box may not be

displayed.  In that case, you won't be able to change the default name for

the output file.

If you run out of disk space while converting a file, a message box will

appear saying that there is a "Hardware error on sound device, or file

error on file device."  Depending on when you run out of space, the box may

appear more than once - just keep hitting <enter>.  Normally, what you will

get in this case is a valid sound file in the format supported by the file

"device," but only the beginning of the original sound will be there.

Sometimes, however, the message means that you have a bad disk or that the

file structure is corrupted (use CHKDSK or SCANDISK to find out).

Note:  If you are using a file "device" to convert between file types, it

may be well to turn off "Enable looping" in the "Defaults" menu.  Some

sound files can loop (play over and over again) - sometimes infinitely.  If

you attempt to convert such a file with looping enabled, it will continue

writing to your disk until it is full.  For that reason, you should listen

to files you want to convert before converting them, and disable looping if

they do not stop.  (Currently, looping is only supported for .voc files.)

Since file "devices" are just device drivers, what was said above about

making new device drivers applies to them too.

BACKING STORE

To help accomodate slow machines, as well as to permit the playing of

compressed file types that require some time to decompress, Dmsound

prepares and loads as much of the file as possible before starting

playback.  In addition to conventional RAM, EMS, XMS, and disk space can be

used to store prepared samples.

You choose the "Storage options..." item in the "Defaults" menu to change

the amount and type of backing store configured.  The amount of backing

store is set in "blocks" of 16 kilobytes; the maximum is 256 (4 megabytes),

and the minimum is 0 (no backing store - use conventional RAM only).  To

permit the use of a particular type of storage, use the spacebar or mouse

to place an "X" in the box for it; conversely, to disable use of that type,

remove the "X".  EMS will be used in preference to XMS, XMS will be used in

preference to conventional RAM, and conventional RAM will be used before

disk space.  If you select a type of storage you don't have, the setting

will be ignored.

If you use disk as backing store, a temporary file named DMSNDTMP.$$$ will

be created in one of three places.  First, if the TEMP environment variable

is defined, DMSNDTMP.$$$ will go in the directory specified by the

variable.  If TEMP is not set, but TMP is, DMSNDTMP.$$$ will go in that

directory.  Finally, if neither variable is set, the temporary file will go

in the root directory of the hard drive with the most free space.

DMSNDTMP.$$$ will automatically be deleted when Dmsound terminates, or if

you turn off the machine while Dmsound is running, it will be deleted the

next time you run Dmsound.

When setting the storage options, you need to bear in mind the issue of

"interrupt latency."  Playing sound is, of course, very time-critical.

Generally, if the soundcard supports DMA it doesn't matter what type of

backing store you use.

If you are playing on a device such as the PC speaker that does not support

DMA, however, you should not use a type of backing store if accessing that

storage leaves interrupts disabled for an extended (measured in

microseconds) period of time.  A hardware EMS card has a low interrupt

latency and should be all right, and a hard drive or floppy drive should be

all right too, provided it uses DMA (and in the case of the floppy drive,

provided it is fast enough to keep up with the soundcard).  When playing on

"synchronous" devices that do not support DMA, you should avoid XMS,

software EMS (such as EMM386), or a hard drive that uses PIO ("processor

I/O") instead of DMA to transfer data between the drive and memory.

Of course, it is up to you, and depending on your hardware, you may not

have a choice if you want to play a particularly long sound file.  A

problem with interrupt latency will manifest itself as a noise every few

seconds while the storage is being accessed.  The same noise can occur if

you have insufficient memory and backing store to preload the whole file,

and it needs to be read from a hard drive using PIO while it is being

played.

Of course, regardless of the amount of backing store configured, some large

files will not fit.  In that case, Dmsound will preload as much of the file

as possible before starting playback, then continue to load the file as it

is being played.  In that way, even if the sound is played faster than it

is loaded, the soundcard may still stay ahead of the loading process, given

the headstart.  If the soundcard does not stay ahead, the sound will

"stutter."  Converting the file to an 8-bit mono type, especially Tandy

.snd, will help that.

If you are using a file "device" to do file conversion, backing store is

not needed.  If you need to convert a large file, it may be good to disable

disk backing store to free up disk space for it.

The less backing store, the faster playback will start, but your machine

needs to be fast enough to keep up.  On fast machines (a 386 can play

Microsoft ADPCM .wav's in real time), you can disable backing store

entirely for fastest playback.

SETTING THE VOLUME

If your soundcard supports setting the volume (of the current drivers, the

Tandy DAC, SoundBlaster Pro, SoundBlaster 16 mono, and SoundBlaster 16

support this), you use the "Set volume..." option in the "Volume" menu to

set it.  You enter the volume as a percentage, where 100% is full volume

and 0% is silence.

You can enter the volume by keyboard or by mouse.  By keyboard, you type

the volume you want in the editfield on the top left.  By mouse, you use

the scrollbar on the right.  There are a pair of radio buttons above the

editfield and the scrollbar to choose between the two methods.  To use the

keyboard method, tab to the radio buttons and press <space> on the button

above the editfield; to use the mouse, click on the button above the

scrollbar.  The radio button setting will be remembered for next time, and

when you save the configuration (see "Saving the Configuration," below),

the radio button setting will be saved.

By keyboard, just type the volume you want into the editfield and press

<enter>.  Pressing <tab> will verify the setting and show it on the

scrollbar.  Press <escape> to leave the volume unchanged.

By mouse, you can click on the scrollbar or drag the elevator to change the

volume.  Clicking on an arrow button changes the volume by 1% in the

direction of the arrow; holding down the button changes it by 1%

repeatedly.  Clicking between the button and the elevator changes the

volume by 10%; again, holding down the button changes it repeatedly.  You

can also drag the scrollbar elevator to the desired percentage.  The

editfield will track the selected percentage.  Click on "OK" when finished,

or "CANCEL" to leave the volume unchanged.

Changing the volume setting by 1% does not necessarily change the volume by

1% - it depends on the device.  The Tandy DAC, for example, supports 8

volume levels; the SoundBlaster 16, 32.

On any output device, you can have Dmsound amplify the samples before

sending them to the device.  To do this, select the "Amplify samples"

option in the "Volume" menu.  The amplification function is fixed, so you

just turn it on or off.  Amplification can add distortion to the sound, so

if your soundcard doesn't have volume control it's better to get a pair of

amplified speakers (about $20 at Radio Shack) and plug them into the

earphone jack on your computer or soundcard, if you have one.

SAVING THE CONFIGURATION

Once you have Dmsound set up for the directory where you keep your sound

files, the amount and type of backing store you want to use, the sound

driver you want, the volume and amplification settings, the assumptions

that should be made about headerless files, and other stuff, you will want

to save that information so you don't have to enter it all again every time

you run the program.  To do that, choose the "Save configuration" option in

the "File" menu.  A file named DMSOUND.CFG will be created in the directory

where DMSOUND.PDM resides.  DMSOUND.CFG is optional, and if you want you

can start over by deleting the file.  More easily, you can just choose

"Save configuration" again after changing the settings you want to change.

In addition to DMSOUND.CFG, Dmsound will create a configuration file for

each sound driver that has selectable settings; this is done automatically.

You can change the settings for the soundcard by using "Select sound

device..." option in the "Defaults" menu to reload it.  The soundcard

configuration files are named after the driver, for example, DMSNDAC.RES

uses DMSNDAC.CFG.  As in the case of DMSOUND.CFG, the soundcard

configuration files are optional and will be created automatically if they

don't already exist, when you select that driver.

For those who have the DeskMate Development Kit, the configuration files

are standard DeskMate configuration files and can be accessed through the

DeskMate environment manager.  Contact the author for information on this.

FOR DESKMATE PROGRAMMERS

If you have the DeskMate Development Kit (SDK), you can use this sound

driver set to add device-independent sound support to your program.  Write

the author if interested.

The source code for Dmsound is available on request.  It requires the SDK

libraries.

ACKNOWLEDGEMENTS

John Ball provided the source code for his Playsnd sound file player for

DOS.  It was a useful starting point for handling several file formats and

provided an example of an excellent user interface.

Guido van Rossum's Audio File Formats FAQ from Usenet clarified some things

and gave further leads.

Mu-law decompression is based on Jeff Poskanzer's code distributed in the

sources for Sox, a public-domain format converter.  Mac HCOM decompression

is from Guido van Rossum's code in Sox.  (Note:  The memory management in

the HCOM decompression module in the DOS version of Sox is broken - only

very short files like the example in the sources can be decompressed.

Write me for a fixed version.)

A-law decompression is based on public-domain source code provided by Sun

Microsystems.

The MSADPCM decompression code is based on an example application for

Windows supplied by Microsoft.

Tandy Corporation supplied the DeskMate Developer's Kit, version 3.03, with

which this program was compiled.  Tandy .snd decompression uses the Tandy

Digital Sound Toolkit.

The SoundBlaster-family drivers were written using Craig Jackson's free

SoundBlaster Programming Information, v0.90.

This program was compiled using Microsoft QuickC, version 2.0, and the

shareware A86 assembler.

OTHER PROGRAMS (BLATANT PLUGS)

If you're running an old IBM PCjr or XT-class Tandy 1000-series with the

Texas Instruments SN76496 3-voice tone and noise generator (but without the

Tandy DAC), Dmsound is not the program you need - get Play3voi instead.

Play3voi is a simple DOS command-line program that plays .wav files on the

3-voice chip.  It is optimized for slow machines and offers 10 levels of

amplification, automatic conversion of stereo and 16-bit files, and

automatic fallback on the sampling rate.  The last version is 0.4; it also

includes a "piano-player" toy for the 3-voice chip.  Source included.

Dmgif is my GIF/JPEG viewer for DeskMate.  IMHO, it is the best viewer

available for Tandy 1000 and pre-VGA systems (up to mono or grayscale VGA).

It is menu- and mouse-driven (though a mouse is not required), provides

online help, and supports panning, rescaling, zooming (several levels), and

color adjustment on all monitors (CGA, EGA, MCGA, VGA, Tandy 1000/PCjr,

Tandy 1000SL/TL/RL, and Hercules).  A DOS runtime is included if you don't

have DeskMate 3, and there's an install program.  The last version is 1.2.

Both of the above (along with other programs and information for the Tandy

1000's) are at my ftp site (see below).

CONTACTING THE AUTHOR

The latest version of Dmsound (and the DOS runtime) can be found at my ftp

site:

ftp.agate.net:/users/01935

If you get "Host not found," try 199.191.1.2.  At this point, Dmsound is in

/users/01935/beta and changes pretty frequently.  When I decide it's more

or less done, it will be in /users/01935/tandy1000/sound.  Play3voi v0.4 is

in that directory, as are John Ball's Playsnd and Sbplay.  Dmgif v1.2 is in

tandy1000/graphics.  There are a lot of files for the Tandy 1000's on the

site (including the Tandy 1000-series FAQ, newest in beta, and all the

setup programs), but we always want more - put them in

/users/01935/incoming.

To contact me, write (postal mail) to:

Jeffrey L. Hayes

130 Forest Ave., Lot 1

Orono, Maine 04473

USA

Or send Internet email to:

tvdog@agate.net

Or you can swear at me personally by telephoning me at:

1-207-866-7903

Finally, if you want to cuss me out publicly :-), I regularly monitor the

Usenet newsgroup comp.sys.tandy.  My news server tends to drop messages,

though, so send a copy by email as well.