%OP%BON

%OP%AMM

%OP%HM3

%OP%FM3

%OP%BM0

%OP%LM10

%OP%HE/@T@

%OP%FO//Z88 Developer's Notes V3, @P@//

%OP%DEError handling and related issues

%CO:A,12,80%%C%%H1%%H2%E r r o r   h a n d l i n g   a n d   r e l a t e d   i s s u e s%H2%%H1%

Before we can look at the details of constructing applications it is necessary

to look at the way in which the Z88 deals with errors. System calls are

accessed by using a Z80 RST instruction followed by one or two bytes which

determine the system function to be accessed. If an error occurs the routine

will return with the Carry of the flags register F (Fc) to be set = 1 and a

return code in register A. The return codes are symbolically represented by

"RC_xxxx", where "xxxx" is a mnemonic for the error. Most errors may be handled

in a straightforward way, with the Carry flag being examined and appropriate

action being taken depending on the value of register A. However, a mechanism

exists to trap errors before they return to the part of code from which they

were called. This mechanism is called an 'error handler' and the programmer may

install one or more of these for use with the application. More than one error

handler could be used if different parts of the application had different error

handling requirements. An error handler will in general ignore most errors,

returning to the code from which the call originated with Fc = 1 and the error

code intact in A, however other errors may be trapped, dealt with and whatever

return code is thought fit can be returned to the running program. The error

handler should perform a RET instruction when it has finished it's work. Note

that this returns control to the operating system and not to the program which

made the original call.

If an error occurs in a system call, then after the call has finished, insofar

as it is able, the error handler is called with Fc = 1, and a return code in A,

and possibly Fz = 1 to indicate a fatal error. Fatal errors, such as RC_HAND,

are so called because the system can take no reasonable action to recover. On

exit from the error handler a fatal error will generate an interactive error

box and following this RC_QUIT will be returned. For non-fatal errors the error

handler may take whatever action it wishes, such as displaying an error

message, but must always return control to the error handler supervisor, with

Fc = 0, by a RET instruction. This will then pass control back to the program

just after the call.

NOTE: The error handler is programmer defined. The reference section of these

notes describes in detail which registers will be affected by system calls on

return to your application. When you install an error handler, possibly other

registers will be affected as well (which you use in the error handler). If you

wish the error handler to act neutral, then preserve the Z80 registers before

executing your error handler, and restore them when the error handler has

finished its work.

Some errors occur because of some action by the user, such as pressing the

<ESC> key or switching between applications. The causes of these kind of errors

are:

1.

2.

RC_SUSP

RC_DRAW

RC_QUIT

RC_ESC

%H2%Pre-emptable Calls%H2%

Only calls which read the keyboard can be pre-empted. Since the keyboard is

usually bound to standard input the most common pre-emptable calls are:

However, if file I/O is bound to the keyboard, then file read operations will

also be pre-emptable. Primarily the suspendable calls are all file I/O and the

delay routine (OS_DLY), but again other calls may be if they take their input

from a stream which is opened to the keyboard device. Remember some re-binding

may be done by the user using the CLI, covered later in "Standard I/O and the

CLI".

%P0%

%H2%System error routines%H2%

The system provides four routines to handle errors:

OS_ERH

OS_ESC

GN_ESP

GN_ERR

An error handling routine may be set up by using the system OS_ERH whose

specification is:

______________________________________________________________________________

OS_ERH, set error handler

RST 20H, DEFB $75

IN:

OUT:

Registers changed after return:

_______________________________________________________________________________

We can now consider the code the error handler should include. Fatal errors may

be ignored (include a RET Z at the start), because there is no reasonable

action to take anyway except termination of this instantiation of the

application. An RC_SUSP error code will generally be completely ignored by the

error handler whereas RC_ESC may want to be acknowledged using OS_ESC, or left

pending for the main part of the application to acknowledge. RC_DRAW could be

responded to by calling a routine to regenerate the screen and then quitting

the error handler with A = RC_SUSP and Fc = 0 as though nothing had ever

happended. Other errors may want to be reported, by outputting an error

message.

Finally, the error handler, rather than the main application, will generally

respond to a KILL request in the form of the RC_QUIT code. It must close

down it's application using OS_BYE, but it is first essential to close open

resources like files, filters, streams and de-allocate memory. If an

application has not properly closed resources, in the form of memory, handles

and buffers, will be lost to the system. Files left open will be permantently

marked as "In Use" and so cannot be deleted or opened for updating. To regain

lost handles and resources to the system, a soft reset must be issued.

%H2%Example%H2%

The following example illustrates the structure of an error handler with a few

responding error codes:

_______________________________________________________________________________

include ":*//errors.def"

include ":*//stdio.def"

inlcude ":*//director.def"

.install

;

; error handler entry:

.error_han

.not_escape

.not_quit

_______________________________________________________________________________

The error handler above responds to an Escape condition and a KILL request. All

other errors are 'ignored' with a display of the system message corrseponding

to the error code. Please note that no registers are changed, except Fc (Carry)

and Fz (Zero) flags. Your error handler might respond to RC_DRAW and issue a

routine to redraw the application windows. Remember that your routines might

change registers that they're not supposed to regarding the outside world of

the error handler.

NOTE: When an error handler is installed, the system relies on having it

available in the corresponding segment (which the address of the error handler

identifies). If you change the bank binding state in the segment of the error

handler, thereby removing the original code, a call to that address by the

system would most likely crash the machine (executing random instructions)!

Further, we suspect a bug in OZ that crashes popdowns if the error handler is

resided in segment 2. Always keep error handlers in segment 3 - all system

applications do it.

%P0%

%CO:B,12,68%

An Escape condition occurs. The <ESC> key has been pressed while

Escape detection is enabled, eg. file transfer calls can be

interrupted by <ESC>. This condition gives rise to the RC_ESC code.

Possibility of pre-emption or suspension. This can happen when the

SQUARE or DIAMOND keys are used, menu or help access has occurred,

the machine has been switched off and back on, or genuine

pre-emption has occurred, ie. the user has switched tasks and

subsequently returned. This condition can give rise to RC_SUSP,

RC_DRAW or RC_QUIT.

Suspicion of pre-emption, the computer was switched off. This code

indicates that the screen is intact, but unsafe workspace may be

changed.

Similar to RC_SUSP except that the application's screen has been

corrupted and the application should attempt to redraw it.

Applications which might find it very difficult to redraw a screen

can ask the operating system to preserve it for them before they

are pre-empted. This can only take place if there is enough memory

to do so - some way of dealing with RC_DRAW is still required.

Note, it is also possible for applications to save a particular

screen image and restore it later - see OS_SR in "Miscellaneous

useful routines".

The process has been pre-empted and a KILL request made for it from

the INDEX (or by own suicide request). The reader may have noticed

from the screen that a KILL request causes the relevant process to

be re-entered momentarily - processes are always expected to kill

themselves off in resonse to this error; they are not killed BY the

INDEX. RC_QUIT may also be returned when after a fatal error has

occurred.

An Escape condition has been detected. This means <ESC> has been

pressed while Escape Detection is enabled. Note the Escape

condition must be acknowledged by OS_ESC, otherwise the Escape

condition will continue to flag errors - acknowledging Escape is an

ideal job for an error handler.

OS_IN

OS_TIN

GN_SIP

OS_SR

OS_GB

OS_GBT

OS_PB

OS_PBT

OS_MV

OS_DLY

set error handler. This call installs a new error handler,

returnning the address of the old handler so that it may be

re-installed at a later date if required.

examine special conditions. This call covers the details of working

with the Escape key, which can generate errors if required.

return a pointer to a system error message. A subset of error codes

have messages associated with them. This call returns an extended

pointer to an error message in the operating system ROM, if it

exists.

display system error box. This draws an error box on screen which

asks the user to press ESCAPE to continue, RC_DRAW or RC_SUSP will

be returned. In the case of fatal errors, eg. RC_BAD or RC_HAND,

the user is asked to press Q to quit and the routine exits with an

RC_QUIT error. The routine always exits with Fc = 1, ie. an error

condition.

A = 0, other values reserved for system use

B = 0

HL = address of new error handler

HL = 0, use default system error handler (in segment 0)

Fc = 0

A = old call level

HL = address of old error handler

...CDE../IXIY same

AFB...HL/.... different

xor  a

ld   b,a

ld   hl,error_han

call_oz(os_erh)

...

ret  z

cp   RC_ESC

jr   nz, not_escape

ld   a, SC_ACK

call_oz(OS_ESC)

ld   a, RC_ESC

call_oz(GN_ESP)

call_oz(GN_SOE)

or   a

ret

cp   rc_quit

jr   nz, not_quit

xor  a

call_oz(os_bye)

or   a

ret

%CO:C,12,56%

get character from standard input

as OS_IN, but with timeout

get line from standard input

used to generate a "Page Wait"

get byte from file or stream

get byte from file or stream, with timeout

put byte to file or stream

put byte to file or stream, with timeout

move bytes between memory and file or stream

delay a certain period

%CO:D,12,44%

; error call/code definitions

; standard input/output definitions

; director call definitions

; zero A

; zero B

; address of error handler

; install new error handler...

; main application goes here...

; error is fatal, so exit back to system

; check for <ESC>

; acknowledge RC_ESC = SC_ACK

; escape processing call

; error code

; get extended address to error message

; write message to standard output

; Fc = 0, Fz = 0

; check for KILL request

; code to close files and de-allocate

; memory goes here...

; no error message on exit...

; force remove application from system

; Fc = 0, Fz = 0

%CO:E,12,32%%CO:F,12,20%%CO:G,12,8%